Application No.:	  376463

Equipment Category – Flexographic Printing Line

| 1. general information |  |  | DATE: 6/27/2001 |
| --- | --- | --- | --- |
| a. manufacturer: PCM/Vision II |  |  |  |
| b. type: 8-color, Web-Fed |  | c. model: 7784 |  |
| d. style: Central impression |  |  |  |
| e. applicable aqmd regulation xi rules: 1130, 1171 |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 5 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 376463 |
| --- | --- | --- | --- |
| A. FUNCTION: Prints plastic packaging/bags (e.g., frozen vegetable packaging)--this qualifies as "Specialty Flexographic Printing" under Rule 1171. |  |  |  |
| b. size/dimension/capacity: 37" |  |  |  |
| c. blowers: |  | d. total flow rate: scfm |  |
| e. material stored/processed/handled: |  |  |  |
| f. throughput/process rate/usage rate: |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 376463 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Lawson Mardon Packaging |  |  |  | B. SIC CODE: 2759 |
| C. ADDRESS: 1120 E. Sandhill Ave. CITY: Carson STATE: CA ZIP: 90746 |  |  |  |  |
| D. CONTACT PERSON: Dan Garvey |  | E. PHONE NO.: 310-631-6170 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 376463 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Bijan Ataian |  |  | D. PHONE NO.: 909-396-2454 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: F36519 ISSUANCE DATE: 1/19/2001 check if no p/c P/O NO.: F36519 ISSUANCE DATE: 1/19/2001 |  |  |  |  |
| F. START-UP DATE: |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: 376463 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: Facility-wide VOC limit of 164 lb/day. VOC emission from this printing unit not to exceed 55 lb/day. Compliance with Rule 1171 (VOC in cleanup solutions not to exceed 810 g/l as applied [600 g/l after 11/30/01]). VOC in inks not to exceed 1.5 lb/gallon less water and exempt solvents. |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: Use of water-base inks with VOC not to exceed 1.5 lb/gal less water and exempt solvents. Use of cleanup solutions containing 0.6 lb/gal (72 g/l) VOC. Drying oven and air heater producing heated air used for drying both to utilize low-NOx burners (see Section III BACT listing for "Dryer or Oven", A/N 376463). |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: Use of water-base inks with VOC not exceeding 1.5 lb/gal less water and exempt solvents was pre-existing BACT. Use of cleanup solutions containing 0.6 lb/gal VOC was suggested by the applicant. |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: |  |  |  |  |  |  |
| B2. TYPE: |  |  |  |  |  |  |
| B3. DESCRIPTION: Low-VOC cleaning solution is made up on site from water:propanol:ammonia in 90:6:4 volumetric ratio. This actually equates to 0.4 lb/gal (48 g/l) VOC. |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: ISSUANCE DATE: P/O NO.: ISSUANCE DATE: |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: ACTUAL CONTAMINANT LOADING: BLOWER HP: |  |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: This printing line is just starting up. However, the low-VOC cleaning solution has been used at this facility for a period well in excess of 6 months. |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: Records review--showed use of cleaning solution containing 0.4 lb/gal VOC for last 6 months of 2000. |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: DATES: CAUSES: |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: 2 DATES: 11/99, 10/00 CAUSES: Inadequate records |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: No source test required CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICEINCY: SOURCE TEST/PERFORMANCE DATA: OPERATING CONDITIONS: TEST METHODS: |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 376463 |
| --- | --- | --- |
| Cleaning solution containing 0.4 lb/gal (48 g/l) VOC is more stringent than Rule 1171 currently requires for Specialty Flexographic Printing (810 g/l) and is also more stringent than the lower Rule 1171 limit for Specialty Flexographic Printing (600 g/l) taking effect 12/1/01. The propanol-water-ammonia cleaning solution works well if cleaning is done before ink begins to harden. |  |  |