Application No.:	  368224

Equipment Category – Lithographic Printing, Heatset and Non-Heatset

| 1. general information |  |  | DATE: 11/21/2003 |
| --- | --- | --- | --- |
| a. manufacturer: Various |  |  |  |
| b. type: Web-fed, heatset and sheet-fed, IR dry |  | c. model: Various |  |
| d. style: |  |  |  |
| e. applicable aqmd rules: 1130, 1171 |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 6 days/wk 50 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 368224 |
| --- | --- | --- | --- |
| A. FUNCTION: This Title V facility has six web-fed printers in one press room and six sheet-fed printers in a second press room. The web-fed presses have natural gas-fired drying ovens, and the sheet-fed presses have IR drying ovens. Printing occurs on both porous and nonporous substrates. Products are publications, brochures, calendars. |  |  |  |
| b. size/dimension/capacity: The sheet-fed printers have 40" sheet width. The web-fed press ovens range from 2.66 to 7.4 MMBtu/hr |  |  |  |
| c. blowers: |  | d. total flow rate: scfm |  |
| e. material stored/processed/handled: |  |  |  |
| f. throughput/process rate/usage rate: |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 368224 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Anderson Lithograph |  |  |  | B. SIC CODE: 2752 |
| C. ADDRESS: 3217 S. Garfield Ave. CITY: Los Angeles STATE: CA ZIP: 90040 |  |  |  |  |
| D. CONTACT PERSON: Frank Barnett |  | E. PHONE NO.: 323-727-0035 x237 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 368224 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Shailesh Patel |  |  | D. PHONE NO.: 909-396-2681 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: ISSUANCE DATE: check if no p/c P/O NO.: F31263 ISSUANCE DATE: 6/1/2000 |  |  |  |  |
| F. START-UP DATE: 1995 |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: 368224 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: Compliance with Rules 1130 (compliance is via afterburner for heatset presses and by compliant inks for IR-dry presses) and 1171 (compliant materials). VOC from facility not to exceed 16,590 lb in any calendar month. VOC in fountain solution not to exceed 0.7 lb/gal including water and exempt solvents. VOC composite partial pressure in blanket and roller washes not to exceed 10 and 6 mm Hg at 20C, respectively. Web press ovens to be vented to an afterburner with minimum overall VOC control efficiency of 98%. Presses to be vented by permanent total enclosure to cogeneration gas turbine when it operates. Overall VOC control efficiency of gas turbine to be at least 90%. |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: Compliance with Rules 1130 and 1171; 0.7 lb/gal VOC in fountain solution including water and exempt solvents; VOC composite vapor pressures in blanket and roller washes at 20C not to exceed 10 and 6 mm Hg, resp.; overall control of VOC from web press ovens 98% or more. |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: Compliance with Rule 1171 and limits on VOC in fountain solution and washes were based on AQMD BACT Guidelines, Part D. 98% overall control of VOC from web press ovens has been achieved in practice. |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Gas turbine (A/N 278856) is Allison model 501-KBS. Afterburner (A/N 346730 is TEC/KATEC model 2-218. |  |  |  |  |  |  |
| B2. TYPE: Gas turbine is combined cycle. Afterburner is equipped with a recuperative heat exchanger. |  |  |  |  |  |  |
| B3. DESCRIPTION: Gas turbine: 48.62 MMBtu/hr input to turbine, 3.757 MW, NOx control by water injection and SCR. NOx and CO limits: 9 and 10 ppmvd@15%O2 with NOx/CO CEMS. Afterburner: 8.936 MMBtu/hr. |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: 278856 ISSUANCE DATE: 5/26/1993 P/O NO.: F37722 ISSUANCE DATE: 3/2/2001 |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 19,130 DSCFM ACTUAL CONTAMINANT LOADING: 185 ppm as C1 BLOWER HP: |  |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: NOx, CO, PM10 |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: 26.5'L x 8.5'H x 7'W |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: The cogeneration system has been in nearly continuous use since it's startup and the presses have been vented to gas turbine whenever it operates. |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: Aileen Santos DATE: 12/19/01, 1/9/03 |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: Inspection |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: DATES: CAUSES: |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: DATES: CAUSES: |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: 3/3/98 and 7/9/1998 CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICIENCY: SOURCE TEST/PERFORMANCE DATA: Turbine 7/9/98 Afterburner 3/3/98 Inlet Outlet Inlet Outlet Temperature, F 82 350 248 475 Flow Rate, DSCFM 19,130 31,600 19,720 19,724 O2, % (dry) 21.0 15.6 20.3 18.6 NMHC ppmvd as C1 185 9 3,181 4 lb/hr as C 6.6 0.53 117.3 0.15 Destruction Efficiency, % 92 99.9 OPERATING CONDITIONS: TEST METHODS: The source test was approved by AQMD Monitoring & Source Test Engineering group. Press rooms were verified to be permanent total enclosures. |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 368224 |
| --- | --- | --- |
| Anderson Lithographic was a recipient of an AQMD Clean Air Award in 2003 for the innovative use of a cogeneration system to control VOC emissions while producing steam and electricity for the plant. The facility is not required to use the cogeneration system at all times, but when it does, it may take credit for the VOC emission reduction. This listing is to encourage other facilities to do the same, not to make this a BACT requirement. |  |  |