south coast air quality management district

Final Environmental Assessment for:

Proposed Amended Rule 1122- Solvent Degreasers

September 21, 2004

SCAQMD No. 040729MK

Executive Officer
Barry R. Wallerstein, D.Env.

Deputy Executive Officer
Planning, Rule Development and Area Sources
Elaine Chang, DrPH

Assistant Deputy Executive Officer
Planning, Rule Development and Area Sources
Laki Tisopulos, Ph.D.

Planning and Rules Manager
Planning, Rule Development and Area Sources
Susan Nakamura

Author:	Michael Krause		Air Quality Specialist

Technical	Rizaldy Calungcagin	Air Quality Engineer II
Assistance:

Reviewed By: 	Steve Smith, Ph.D.	Program Supervisor

William Wong		Senior Deputy District Counsel

Louis Yuhas		Air Quality Analysis and Compliance Supervisor

South coast air quality management district

governing board

Chairman:	WILLIAM A. BURKE, Ed.D.

Speaker of the Assembly Appointee

Vice Chairman:	S. ROY WILSON, Ed.D.

Supervisor, Fourth District

Riverside County Representative

MEMBERS:

MICHAEL D. ANTONOVICH

Supervisor, Fifth District

Los Angeles County Representative

JANE CARNEY

Senate Rules Committee Appointee

WILLIAM CRAYCRAFT

Councilmember, City of Mission Viejo

Cities Representative, Orange County

BEATRICE J.S. LAPISTO-KIRTLEY

Mayor, City of Bradbury

Cities Representative, Los Angeles County, Eastern Region

RONALD O. LOVERIDGE

Mayor, City of Riverside

Cities Representative, Riverside County

JAN PERRY

Councilmember, City of Los Angeles

Cities Representative, Los Angeles County, Western Region

BILL POSTMUS

Supervisor, First District

San Bernardino County Representative

JAMES SILVA

Supervisor, Second District

Orange County Representative

CYNTHIA VERDUGO-PERALTA

Governor's Appointee

DENNIS YATES

Councilmember, City of Chino

Cities Representative, San Bernardino County

EXECUTIVE OFFICER:

BARRY R. WALLERSTEIN, D.Env.

Table of contents

CHAPTER 1 - PROJECT DESCRIPTION

Introduction 		1-1

Legislative Authority		1-2

California Environmental Quality Act		1-3

Project Location		1-4

Project Background		1-5

Project Objectives		1-12

Project Description		1-12

Emission Inventory		1-14

CHAPTER 2 - ENVIRONMENTAL CHECKLIST

Introduction			2-1

General Information		2-1

Environmental Factors Potentially Affected		2-2

Determination		2-2

Environmental Checklist and Discussion		2-4

APPENDIX A - Proposed Amended Rule 1122

List of Figures

Figure 1-1: South Coast Air Quality Management District		1-4

List of Tables

Table 1-1: Facilities Using the Rule 1122 Exemption, Primary Operations
and Solvents Used		1-6

Table 1-2: Emissions Inventory of Small Batch-Loaded Cold 
Cleaners and Vapor Degreasers		1-14

Table 2-1: Air Quality Significance Thresholds		2-8

Table 2-2: Construction Emissions from Installing Control Equipment 		2-10

Table 2-3: Estimated Operational Emissions from Thermal Oxidizers		2-11

Table 2-4: Anticipated Foregone VOC Emission Reductions 		2-14

Table 2-5: Total Projected Natural Gas Usage from Thermal
Oxidizer Operations		2-19

Preface

The Draft Environmental Assessment (EA) for the proposed amendments to Rule 1122 – Solvent Degreasers was circulated for a 30-day public review and comment period from July 29, 2004 to August 27, 2004.  One public comment letter was received that directly comments on the Draft EA and responses to the comments were included in the Final EA.  Minor modifications were made to the Draft EA so it is now a Final EA.  Deletions and additions to the text of the EA are denoted using strikethrough and underlined, respectively.  Changes to the project description and potential impacts from the changes were evaluated and do not alter the conclusions made in the Draft EA or worsen environmental impacts analyzed in the Draft EA.  Pursuant to CEQA Guidelines §15073.5(c)(2), recirculation is not necessary since the information provided does not result in new avoidable significant effects.

C H A P T E R   1  -  P R O J E C T   D E S C R I P T I O N

Introduction

Legislative Authority

California Environmental Quality Act

Project Location

Project Background

Project Objectives

Project Description

Emission Inventory

# Introduction

Volatile organic compound (VOC) emissions are major contributors to the formation of ozone (key ingredient for smog formation) in the South Coast Air Basin (Basin).  The formation of ozone occurs as VOCs react with oxides of nitrogen (NOx) in the atmosphere.  Ozone, a criteria pollutant, has been shown to adversely affect human health.  It also contributes to the formation of another criteria pollutant, particulate matter with a diameter less than 10 microns (PM10) and with a diameter less than 2.5 microns (PM2.5).

South Coast Air Quality Management District (SCAQMD) Rule 1122 – Solvent Degreasers, was adopted on March 2, 1979, to control VOC emissions from solvent degreasing operations.  The rule establishes both equipment and operating requirements for any type of solvent degreasing operation.  Industries subject to the provisions of Rule 1122 include any facility that operates degreasing equipment that removes contaminants as part of its production process.  Solvent degreasing involves the use of solvents, in either liquid or vapor phase, to remove contaminants such as dirt, oil, soil, and grease from parts, products, tools, machinery, and equipment.

The 1997 amendments to Rule 1122 required that cleaning materials used in cold cleaning operations contain no more than 50 grams per liter of VOC.  The 1997 amendments also included a limited exemption that allowed the continued use of high VOC solvents until January 1, 2003 for small batch-loaded cold cleaners and vapor degreasers for specific types of cleaning applications.  These cleaning applications included electrical, high precision optics or electronics applications; or aerospace and military applications for cleaning solar cells, laser hardware, space vehicle components, fluid systems and components used solely in research and development programs, or laboratory tests in quality assurance laboratories.  The solvent used in this equipment is limited to less than five gallons per calendar month.

In 2001 amendments to Rule 1122 was further amended to reduce the VOC content limit to 25 grams per liter for cold cleaning materials used in cleaning operations by January 1, 2003.  In 2002, the exemption for small batch-loaded cold cleaners and vapor degreasers was extended for two years from January 1, 2003 to January 1, 2005.  Further, the rule required a technology assessment by year 2004 to determine whether to retain the exemption of small batch-loaded cold cleaners and vapor degreasers from the requirements of Rule 1122.

The Preliminary Staff Report and 2004 Technology Assessment for Small Degreaser Exemption Under Rule 1122 – Solvent Degreasers (SCAQMD, June, 2004) concluded that most military and aerospace contractors utilize aqueous and/or exempt solvent cleaners as part of their cleaning process.  Many of these companies have successfully converted to alternative solvents for cleaning applications involving high-precision optics, laser hardware, electrical applications, and fluid systems.  Others remain reluctant to convert.  Acetone, aqueous solutions, and soy-based cleaners are examples of alternative cleaners.  Cleaning of high-reliable electronic components and space vehicle components using low VOC cleaners, however, has not been completely successful at this point.  Companies engaged in such cleaning activities continue to use VOC solvents until acceptable replacements are found.  The use of an emission control system for a degreasing operations utilizing small-sized degreasers may be a viable compliance option for these types of cleaning activities.

The technology assessment recommends amending the rule to allow for continued use, beyond January 1, 2005, of degreasers with open-top surface areas less than one square foot, or with a capacity less than two gallons only for certain applications, provided such degreasers are vented to a VOC emission control system capable of collecting at least 90 percent, by weight, of the emissions generated by the solvent degreaser and a destruction efficiency of at least 95 percent by weight.  In addition, the technology assessment recommends a permanent exemption be established for small-sized degreasers used for research and development programs, or laboratory tests in quality assurance laboratories, as well as an exemption for batch-loaded cold cleaners and vapor degreasers with open-top surface areas less than one square foot, or with a capacity of less than two gallons used only to clean electronic parts designed to travel over 100 miles above the earth’s surface.  Further, the exemption for photocurable resins from stereolithography equipment and models will be extended to 2008.

Pursuant to the California Environmental Quality Act (CEQA) (California Public Resources Code § 21000 et seq.), this Environmental Assessment includes an analysis of the potential adverse environmental impacts of implementing proposed amended Rule (PAR) 1122.  The environmental analysis determined that a potential adverse impact to air quality would result from foregone VOC emission reductions due to primarily from permanently exempting specific degreasing applications, but concluded that the impact would not exceed the SCAQMD's significance thresholds and therefore is considered not significant.  No other environmental topic areas were identified that could be significantly adversely affected by the proposed amended rule.

# LEGISLATIVE AUTHORITY

The California Legislature created the SCAQMD in 1977 (Lewis-Presley Air Quality Management Act, California Health and Safety Code §§ 40400 et seq.) as the agency responsible for developing and enforcing air pollution control rules and regulations in the South Coast Air Basin and portions of the Salton Sea Air Basin and Mojave Desert Air Basin.  By statute, SCAQMD is required to adopt an air quality management plan (AQMP) demonstrating compliance with all state and federal ambient air quality standards for the district [California Health and Safety Code § 40460(a)].  Furthermore, SCAQMD must adopt rules and regulations that carry out the AQMP [California Health and Safety Code, § 40440(a)].  The 2003 AQMP concluded that major reductions in emissions of VOCs and NOx are necessary to attain the air quality standards for ozone and PM10.  Rule 1122 was originally adopted and subsequently amended to carry out these mandates.  The currently proposed project is based on a technology assessment required as part of the previous amendments to Rule 1122.

# CALIFORNIA ENVIRONMENTAL QUALITY ACT

The proposed amendments to Rule 1122 are a "project" as defined by CEQA (California Public Resources Code §21080.5).  SCAQMD is the lead agency for the proposed project and has prepared appropriate environmental analysis pursuant to its certified regulatory program (SCAQMD Rule 110).  California Public Resources Code §21080.5 allows public agencies with regulatory programs to prepare a plan or other written document in lieu of an environmental impact report (EIR) once the Secretary of the Resources Agency has certified the regulatory program.  The SCAQMD’s regulatory program was certified by the Secretary of the Resources Agency on March 1, 1989, and is codified as SCAQMD Rule 110.

CEQA requires that the potential adverse environmental impacts of proposed projects be evaluated and that feasible methods to reduce or avoid significant adverse environmental impacts of these projects be identified.  To fulfill the purpose and intent of CEQA, the SCAQMD has prepared this environmental assessment (EA) to address the potential adverse environmental impacts associated with the proposed amendments to Rule 1122.  This Draft EA is intended to: (a) provide the lead agency, responsible agencies, decision makers and the general public with detailed information on the environmental effects of the proposed project; and, (b) to be used as a tool by decision makers to facilitate decision making on the proposed project.

All comments received during the public comment period on the analysis presented in the Draft EA will be responded to and included in the Final EA.  Prior to making a decision on the proposed amendments, the SCAQMD Governing Board must review and certify the EA as providing adequate information on the potential adverse environmental impacts of the proposed amended rule.

SCAQMD’s review of the proposed project shows that the project would not have significant adverse effects on the environment.  Therefore, pursuant to CEQA Guidelines §15252, no alternatives or mitigation measures are included in this Draft EA.  The analysis in Chapter 2 supports the conclusion of no significant adverse environmental impacts.

# project location

PAR 1122 would apply to the SCAQMD’s entire jurisdiction.  The SCAQMD has jurisdiction over an area of 10,473 square miles (referred to hereafter as the district), consisting of the four-county South Coast Air Basin (Basin) and the Riverside County portions of the Salton Sea Air Basin (SSAB) and the Mojave Desert Air Basin (MDAB).  The Basin, which is a subarea of the SCAQMD’s jurisdiction, is bounded by the Pacific Ocean to the west and the San Gabriel, San Bernardino, and San Jacinto Mountains to the north and east.  The 6,745 square-mile Basin includes all of Orange County and the nondesert portions of Los Angeles, Riverside, and San Bernardino counties.  The Riverside County portion of the SSAB and MDAB is bounded by the San Jacinto Mountains in the west and spans eastward up to the Palo Verde Valley.  The federal nonattainment area (known as the Coachella Valley Planning Area) is a subregion of both Riverside County and the SSAB and is bounded by the San Jacinto Mountains to the west and the eastern boundary of the Coachella Valley to the east (Figure 1-1).

Figure 1-1

South Coast Air Quality Management District

# PROJECT BACKGROUND

## Solvent Degreasing Process

Solvent degreasing can be conducted as either a batch or conveyorized operation.  With each of these methods, the solvent can be used in either the liquid or vapor state.  Batch-loaded cold cleaners are batch-operated degreasers that are designed to contain liquid solvent, and are always operated at a temperature below the solvent’s boiling point.  The part to be cleaned is lowered into and raised from the bath, and allowed to drain and dry.  The cleaning process can be facilitated by the use of agitation or solvent spray.

When the solvent is used as a vapor (vapor degreasing), the hot vapors condense on the cold article, transferring the dirt and grease to the solvent.  When the article reaches the temperature of the vapor, no further condensation occurs.  The clean article dries and is removed from the degreaser.  This vapor cleaning process has been used for many extensive and difficult cleaning operations.

Many industrial facilities use various solvents for cleaning and degreasing.  Facilities that use VOC-containing solvents fall under the provisions of Rule 1122.  They vary from small users to major manufacturing operations, such as aerospace operations, that may have very sophisticated cleaning and degreasing facilities.  Among the small users are machine shops, which typically use batch-loaded cold cleaners for their degreasing operations.

Rule 1122 currently exempts small batch-loaded cold cleaners and vapor degreasers less than five gallons per month usage for electrical, high precision optics or electronics applications; or aerospace and military applications for cleaning solar cells, laser hardware, space vehicle components, fluid systems; or components used solely in research and development programs, or laboratory tests in quality assurance laboratories.  Facilities affected by the existing rule exemption typically perform an immersion cleaning method for their cleaning operations.  Immersion cleaning refers to dipping or soaking of the parts to be cleaned in a liquid bath (usually organic solvent) at room temperature with no agitation.  The cleaning effectiveness of an organic solvent relies on three fundamental solvent properties: chemical solvency, polarity of the solvent and surface tension.  Chemical solvency indicates the dissolving power of the solvent, the polarity is the effectiveness of the cleaning solvent to remove a particular type of soil or contaminant and surface tension of the solvent determines the penetration of the solvent which is important when small gaps are part of the geometry of the part being cleaned.  The cold cleaning activities subject to the existing exemption are referred to as precision cleaning activities because of the specified level of particle contamination after cleaning.  Different specifications are written for different manufactured parts.  Although commercial components may have different cleanliness specifications, it is generally the military and National Aeronautics Space Administration (NASA) specifications that result in more demanding levels of cleanliness.

The current cleaning practices of the affected facilities are relatively unchanged since the 1997 and 2002 rule amendments.  However, some aerospace companies use water and waterborne chemistries to clean a variety of metal housing and circuit boards.

## Affected Facilities

There are few industries that utilize the limited exemption of small batch-loaded cold cleaners and vapor degreasers from the requirements of Rule 1122.  The types of industries using small batch-loaded cold cleaners and vapor degreasers include aerospace, military, electronics and communications manufacturing of parts for aerospace and military applications.  Parts being cleaned by these industries include high-technology satellite circuit boards, microelectronics, hybrid circuit boards, and high precision optics.  Table 1-1 provides a list of the 13 known facilities that use small batch-loaded cold cleaners and vapor degreasers, including the types of parts being cleaned, types of contaminants being removed and the cleaning solvents used.

Table 1-1

Facilities Using the Rule 1122 Exemption, Primary Operations and Solvents Used

| Affected Facilities | Primary Operation | Types of Contaminants Removed | VOC Solvents Used Under the Exemption |
| --- | --- | --- | --- |
| Boeing Satellite Systems | Satellite circuit boards; microelectronics | Rosin flux; plating residues; flux activators; particulates; oils | Isopropyl Alcohol (IPA) |
| Raytheon | Military microcircuit boards; high precision optics | Rosin flux; plating residues; flux activators; particulates; oils | IPA; Ethanol; Hexane |
| Northrop Grumman Space Technology (formerly TRW) | Space research and development; solar cells; electronics; housing for electronic chips | Rosin flux; plating residues; flux activators; particulates; oils | IPA; IPA/Methylene Chloride; Methanol |
| Teledyne Electronic Technologies | Microelectronics; custom hybrid circuit boards; fiber optics | Rosin flux; plating residues; flux activators | IPA; IPA/Cyclohexane; n-Propyl Acetate; n-Methyl Pyrrolidone |
| Union Technology Corp. | Capacitors | Rosin flux; plating residues; flux activators | Ultrasonic Ethyl Acetate; IPA rinse |

Table 1-1 (concluded)

Facilities Using the Rule 1122 Exemption, Primary Operations and Solvents Used

| Affected Facilities | Primary Operation | Types of Contaminants Removed | VOC Solvents Used Under the Exemption |
| --- | --- | --- | --- |
| Newport Optics | Prisms; lenses; mirrors; windows | Polishing and lapping compounds; pitch | Acetone/n-Propyl Bromide |
| Rand Precision Optics | Prisms; lenses; mirrors; windows | Polishing and lapping compounds; pitch | Citrus-based Cleaner |
| Matrix Systems | Circuit boards | Rosin Flux | Ultrasonic IPA |
| Southern Electronics Co. | Capacitors | Rosin flux | IPA |
| Customs Suppression | Capacitors | Rosin flux | IPA |
| Sota Corp | Fuel pump; circuit board | Rosin flux | IPA |
| CommOptics | Prisms; lenses; mirrors | Polishing compound; pitch | IPA |
| Rolyn Optics | Prisms; lenses; mirrors | Polishing compounds; pitch; wax | Citrus-based cleaner |

## Technical Assessment

SCAQMD staff has conducted site visits (8) and telephone surveys (20) to facilities affected by the exemption to determine progress made in testing alternative cleaning methods and solvents that comply with the 25 gram per liter VOC limit in Rule 1122.  A technical panel, comprised of representatives from various industries directly affected by the exemption, was formed to assist SCAQMD staff in reviewing existing cleaning technologies as well as evaluating the need for the exemption for those cleaning applications identified in section (k)(1)(D).  The Preliminary Staff Report and 2004 Technology Assessment for Small Degreaser Exemption Under Rule 1122 – Solvent Degreasers (SCAQMD, June, 2004) updates the 2002 Technology Assessment and presents new information obtained from SCAQMD staff’s fieldwork as well as input from the Technical Panel on available cleaning materials and technologies.  In addition, data from research studies conducted by the Institute for Research and Technical Assistance (IRTA), an SCAQMD contractor, are also incorporated in the Technology Assessment report.  The report concludes with a recommendation regarding the small-sized degreaser exemption.  SCAQMD Staff has identified cleaning applications currently exempt under section (k)(1)(D) of Rule 1122 where aqueous or low-to-no-VOC cleaning materials have successfully replaced high-VOC solvents.  These applications are discussed in the succeeding paragraphs.

#### Optics

High precision optics refers to electro-optical devices that include laser optics and infrared sensors or detectors used primarily in military and aerospace applications and fiber optics.  Most high precision optics companies have converted to aqueous and/or VOC-exempt solvent (acetone) cleaning.  The removal of wax and pitch presents the biggest challenge to aqueous cleaning of optics.  Anti-reflective coatings are applied after cleaning the glass components, thus, the level of cleanliness is very important with regard to the quality of the applied coating.

#### Electronics

According to studies of the affected industry, an aerospace company that manufactures hydraulic braking systems for aircraft, switched from vapor degreasing using 1,1,1-trichloroethane to water-based cleaning in its printed circuit board defluxing operation.  This company must adhere to a military specification that requires the use of rosin flux.  Rosin flux is removed in a dishwasher-type water system (impingement cleaning) using an aqueous saponifier for flux residue removal.  The cleaning system operates on a 50-minute cycle that involves washing, rinsing and drying.  Other companies are also using water to clean rosin flux from printed wiring electronic assemblies.  It is typically a three stage cleaning process (wash, rinse, and dry) that is identical to the conventional aqueous cleaning process and contains no VOCs.

#### Electrical Applications

Electrical applications include relay switches, transformers and capacitors.  There is consensus among the aerospace companies that water cleaners should not be used for cleaning powered-up components.  However, companies that manufacture electrical switching and control devices (relay switches) for the aerospace industry have been using detergents and water.  One company has experienced difficulty in drying the parts.  Without complete drying, "short outs" may occur when energized, which is the problem with water and polar solvent in general.  However, other exempt solvents such as HCFC-225 in a vapor degreaser can be used in lieu of aqueous cleaning.  Another company that manufactures relay switches currently uses pure AK-225 (HCFC-225) solvent for cleaning relays in a vapor degreaser.  Another company successfully uses water-based chemistries for cleaning the capacitors, with centrifuge drying.

Cables used to power circuit boards have also been cleaned with water.  However, water causes wicking (capillary action) effects particularly on female connectors.  Residual fluids remain inside the cables even after blowing air into the holes for drying.  Testing also indicates corrosive effects associated with the use of water.  Masking has been tried but was found not to be completely effective.  To resolve the problem, one company uses a mixture of acetone and isopropyl alcohol for flux removal in hand-wipe cleaning application.  Another company uses de-ionized water to rinse the capacitors, which are then placed in an oven for subsequent drying.

Overall, however, the data obtained on electrical cleaning applications indicate that alternative cleaning technologies are available for such cleaning applications; therefore, the rule exemption for this specific application is no longer needed.

Compliance Options

#### Organic Solvents

The organic solvents commonly used under the small batch-loaded cold cleaner exemption are alcohols (IPA, ethanol, methanol, etc.) and blends of IPA with cyclohexane.  Cleaning with organic solvents is often faster than with water and is perceived to be a more reliable mechanism for precision cleaning.  Alcohols are often used to keep parts dry after water-based cleaning due to its affinity with water molecules and as cleaners in precision applications.  Alcohols are commonly used in the removal of ionic particles such as electroplating salts and rosin activators used during the manufacture of military and aerospace circuit boards.  Alcohols are also good rinsing agents for other hardware.  Furthermore, alcohols remove oils and fingerprints.  Cyclohexanes or other non-polar solvents are often added to IPA to boost its cleaning effectiveness.

Mirrors, prisms, windows and lenses used in the high precision optics industry undergo certain lapping and polishing procedures to obtain the desired shapes and improve optical characteristics.  In order to hold the parts in place during the process, mounting blocks, wax and pitch are used.  N-methyl pyrrolidone and n-propyl bromide are used to remove the blocking compounds, wax and pitch off the mirrors, prisms, windows and lenses.

#### Aqueous Systems

Water or aqueous systems (washing, rinsing, and drying) are employed in a wide variety of precision cleaning operations.  These aqueous systems replace beakers, buckets and trays of solvents.  The choice of aqueous solutions and additives depends on the type of metal to be cleaned and the type of soil to be removed.  Alkaline aqueous cleaners are the most commonly used because the high pH allows the removal of a variety of soils and oils.  Alkaline cleaners contain additives such as surfactants/detergents, inhibitors, sequestering agents, emulsifiers, and/or saponifiers to aid in cleaning.  Drying of the component is also necessary to aid in the prevention of water spots, corrosion, and other residues.  Pure water can also be used to remove surface contamination typically through spray mechanisms or steam cleaning.

#### Miscellaneous Cleaning Solvents

Soy-based cleaners, a relatively newcomer to the industry, are also being used to remove certain contaminants.  Such cleaners, composed of methyl esters, demonstrate good performance, and are considered safe and cost-effective alternative solvents.  The cleaner provides good solvency, has a very low VOC content and a high flash point, and is compatible with most surface materials and other organic solvents.  Methyl esters are currently being used in optics manufacturing to remove pitch and wax.  However, this solvent is not expected to be a viable replacement in high precision electronics cleaning because of its slow evaporation rate resulting in oily residue on the substrate being cleaned.

Water-based cleaners, VOC-exempt compounds, soy-based products and other alternative cleaning techniques, such as CO2 snow, plasma etch, and laser ablation, are examples of low to no VOC cleaning systems.  Each of these options (as with any organic compounds or blends) should undergo a case-by-case study when evaluating the potential for conversion to a particular cleaning application.

#### Alternative Emission Control Systems

While low-VOC content cleaning materials have been identified for most of the cleaning applications covered by the exemption, there is a preference for using an alternative emission control system, in conjunction with the use of small-sized degreasers as a compliance option in Rule 1122.  The option would allow the flexibility of choosing the most suitable high-VOC cleaning technology for a particular cleaning application, and at the same time, meet the emission reduction requirements of the rule.

Based on information obtained from operators of affected facilities, the two most likely alternative emission control systems expected to be used are an afterburner, either a new one or tied into an existing one, or ductless fume hoods equipped with carbon filters to reduce emissions from the facility’s solvent cleaning operations.

#### Thermal Oxidizers

There are three main categories of thermal oxidizers that could be used to control VOCs: afterburners with no heat recovery, thermal oxidizers with recuperative heat recovery and highly efficient regenerative heat recovery oxidizers.  When thermal oxidizers are used to destroy halogenated organic compounds, special materials or construction are often required, such as fiber-reinforced plastic (FRP) or stainless steel.  In addition, a downstream scrubber is frequently needed to minimize releases of halogenated acid gases.  The extent and type of these additional items depend upon the level of the halogenated compounds in the inlet stream and applicable regulatory requirements.  The following paragraphs briefly describe the three types of thermal oxidizers.

Afterburners: Afterburners are most commonly used to control intermittent and emergency releases of VOCs.  Due to factors such as noise and the lack of heat recovery, (which results in high energy consumption and high NOx and CO2 emissions) their use for steady-state control of VOCs is not widespread.  They are most often used for controlling intermittent releases of ethylene oxide from medical or food product sterilizers.  Afterburners operate in the 1,200 oF to 1,400 oF range with a residence time of at least 0.3 seconds and DREs of 95 to 98 percent.

Both recuperative or regenerative thermal oxidation systems generally consist of a refractory-lined chamber, one or more burners, a temperature-control system and heat-recovery equipment.  Contaminated gases are collected by an industrial ventilation system and delivered to the preheater inlet, where they are heated by indirect contact with the hot oxidizer exhaust.  Gases are then mixed thoroughly with the burner flame in the upstream portion of the unit, and then pass through the combustion zone where the combustion process is completed.  The VOC concentrations in most industrial process vent-streams are too low for self-sustaining combustion.  Therefore, a supplemental fuel (natural gas) is required.  Depending on the heat recovery efficiency, this supplemental fuel requirement may or may not translate into significant annual operating costs.

Recuperative thermal oxidizers: Recuperative thermal oxidizers recover 60 to 80 percent of the system's energy demands with a shell and tube type heat exchanger.  Recuperative units operate in the 1,400oF to 1,600oF range with a residence time of at least 0.5 seconds and DREs of 98 to 99 percent.  Thermal oxidizers with recuperative heat exchangers can recover 80 to 95 percent of the energy requirement.  These recuperative thermal oxidizers use a ceramic medium for heat transfer, which is stored in three or more dedicated beds that feed a central combustion chamber.  Valves control which bed is being preheated by exhaust gases and which bed is transferring its heat to incoming VOC contaminated air.

Regenerative thermal oxidizers:  Regenerative units operate in the 1,800 oF to 2,000 oF range with a residence time of at least 0.8 seconds and DREs of 99 to 99.9 percent.  Regenerative oxidizers cost more than recuperative designs of equal capacity.  However, their life-cycle costs are less because annual fuel costs are less than for recuperative units.

#### Ductless Fume Hoods

Ductless fume hoods are self-contained carbon filtered enclosures that remove hazardous fumes, vapors and particles from most laboratory, industrial and/or commercial applications.  A negative-pressure system draws exhaust fumes upwards from the work area through an exhaust purification filter to allow safe indoor release.  The purification filter is typically an activated carbon filter which acts as an adsorption system of VOC’s in the air stream.

# Project Objectives

The objectives of PAR 1122 are to:

- modify Rule 1122 to implement the recommendations made in the The Preliminary Staff Report and 2004 Technology Assessment for Small Degreaser Exemption Under Rule 1122 – Solvent Degreasers (SCAQMD, June, 2004) which include:

- allow use of alternative emission control system the equipment is used only for cleaning high-precision optics, electrical or electronic components; or aerospace and military applications for cleaning solar cells, laser hardware, fluid systems, and space vehicle components;

- exempt small batch-loaded cold cleaners and vapor degreasers used solely for research and development programs, or laboratory tests in quality assurance laboratories;

- exempt small batch-loaded cold cleaners and vapor degreasers used only for cleaning electronic parts that are designed to travel over 100 miles above the earth's surface.

# PROJECT DESCRIPTION

The following bullet points summarize the main components of PAR 1122.  For a complete description of PAR 1122, the reader is referred to Appendix A.

- Add a definition for “electronic component.”

- Permanently extend the existing exemption from the requirements of Rule 1122 of small batch-loaded cold cleaners and vapor degreasers provided the following five provisions are met:

- the equipment is used only for cleaning high-precision optics, electrical or electronic components; or aerospace and military applications for cleaning solar cells, laser hardware, fluid systems, and space vehicle components; and

- an emission collection and control system is installed and operated which shall collect at least 90 percent, by weight, of the emissions generated by the degreasing operation and have a destruction efficiency of at least 95 percent, by weight; and

- National Emission Standard for Hazardous Air Pollutant (NESHAP) halogenated solvents are not used;

- the equipment is operated in accordance with applicable work practice requirements; and

- the operator meets the monitoring, recordkeeping, and reporting requirements.

- Add an Maintain the existing exemption from the requirements of Rule 1122 for of small batch-loaded cold cleaners and small (less than one square foot) vapor degreasers used only for cleaning electronic parts that are designed to travel over 100 miles above the earth's surface, contains no NESHAP halogenated solvent, and VOC emissions from the equipment do not to exceed 22 pounds per month per facility.  Further, the total VOC emissions from all of the small-sized degreasers used for this type of cleaning are limited to 22 pounds per month per facility.  However, facilities that consolidate their operations after the date of rule adoption will be allowed higher VOC emission limits in accordance with the following guidelines:

  - For two facilities that consolidate at least 85 percent of each of their total VOC emissions from all of their equipment subject to this exemption to one consolidated facility, the VOC emission limit may be increased to 44 pounds total per month for the two consolidating facilities.

  - For more than two facilities consolidating at least 85 percent of each of their total VOC emissions from all of their equipment subject to this exemption to one consolidated facility, the VOC emission limit may be increased to 66 pounds total per month for the consolidating facilities.

Facilities that consolidate their operations must demonstrate to the SCAQMD that the facilities involved in the consolidation are under common ownership with the consolidated facility, and that any applicable permits for the equipment being consolidated has been cancelled by the SCAQMD.  In addition, the facilities must obtain a written concurrence of the consolidation from the SCAQMD specifying the applicable VOC emission limit under this exemption.  The combined VOC emissions from the facilities involved in the consolidation process cannot exceed the applicable monthly limit provided in subsection (k)(1)(E)(ii) for the consolidating facilities.

- Permanently maintain the existing exemption from the requirements of Rule 1122 of small batch-loaded cold cleaners and vapor degreasers used solely for research and development programs, or laboratory tests in quality assurance laboratories.

- Extend the exemption for the cleaning of photocurable resins from stereolithography equipment and models to December 31, 2008.

- Provide the test method to be used to determine the capture efficiency of the emission control system.

- Delete outdated rule language, such as compliance dates which have passed and associated requirements that are no longer in effect.

# Emission INventory

The VOC emissions associated with small batch-loaded cold cleaners and vapor degreasers are low because spent solvents are recycled and because the current work practice is to cover the containers during and after soaking the part.  The companies that utilize the exemption for small batch-loaded cold cleaners and vapor degreasers are listed in Table 1-2, along with their most current actual daily VOC emissions.

Table 1-2

Emissions Inventory of Small Batch-Loaded Cold Cleaners and Vapor Degreasers

| Affected Facilities | Estimated Daily VOC Emissions (pounds per day) |
| --- | --- |
| Boeing Satellite Systems | 5 |
| Raytheon | 3 |
| Northrop Grumman Space Technology | 1 |
| Teledyne Electronic Technologies | 2 |
| Union Technology Corp. | 5 |
| Newport Optics | 2 |
| Rand Precision Optics | 2 |
| Matrix Systems | 0.5 |
| Southern Electronics Co. | 1 |
| Customs Suppression | 0.2 |
| Sota Corp | 0.3 |
| CommOptics | 1 |
| Rolyn Optics | 2 |
| TOTAL ESTIMATED DAILY VOC EMISSIONS from Small Batch-Loaded Cold Cleaners and Vapor Degreasers | 25 pounds VOC per day |

C H A P T E R   2  -  E N V I R O N M E N T A L   C H E C K L I S T

Introduction

General Information

# Environmental Factors Potentially Affected

Determination

Environmental Checklist and Discussion

# INTRODUCTION

The environmental checklist provides a standard evaluation tool to identify a project's adverse environmental impacts.  This checklist identifies and evaluates potential adverse environmental impacts that may be created by the PAR 1122 – Solvent Degreasers.

# GENERAL INFORMATION

| Project Title: | Proposed Amended Rule 1122 – Solvent Degreasers |
| --- | --- |
| Lead Agency Name: | South Coast Air Quality Management District |
| Lead Agency Address: | 21865 Copley Drive Diamond Bar, CA 91765 |
| CEQA Contact Person: | Michael A. Krause (909) 396-2706 |
| Rule Contact Person: | Rizaldy Calungcagin (909) 396-2315 |
| Project Sponsor's Name: | South Coast Air Quality Management District |
| Project Sponsor's Address: | 21865 Copley Drive Diamond Bar, CA 91765 |
| General Plan Designation: | Not applicable |
| Zoning: | Not applicable |
| Description of Project: | The SCAQMD is proposing to amend Rule 1122 to allow the continued use, beyond January 1, 2005, of degreasers with open-top surface areas less than one square foot, or with a capacity less than two gallons only for certain applications, provided such degreasers are vented to a VOC emission control system capable of collecting at least 90 percent, by weight of the emissions generated by the solvent degreaser and a destruction efficiency of at least 95 percent by weight. Further, PAR 1122 establishes a permanent exemption for small-sized degreasers used for research and development programs, or laboratory tests in quality assurance laboratories, and exempts batch-loaded cold cleaners and vapor degreasers with open-top surface areas less than one square foot or with a capacity of less than two gallons used for cleaning electronic parts designed to travel over 100 miles above the earth’s surface. |
| Surrounding Land Uses and Setting: | Not applicable |

| Other Public Agencies Whose Approval is Required: | Not applicable |
| --- | --- |

# environmental factors POTENTIALLY Affected

The following environmental impact areas have been assessed to determine their potential to be affected by the proposed project.  None of the environmental topics are expected to be adversely affected by the proposed project.  An explanation relative to the determination of impacts can be found following the checklist for each area.

|  | Aesthetics |  | Geology and Soils |  | Population/ Housing |
| --- | --- | --- | --- | --- | --- |
|  | Agricultural Resources |  | Hazards and Hazardous Materials |  | Public Services |
|  | Air Quality |  | Hydrology and Water Resources |  | Recreation |
|  | Biological Resources |  | Land Use and Planning |  | Solid/Hazardous Waste |
|  | Cultural Resources |  | Mineral Resources |  | Transportation/Circulation. |
|  | Energy |  | Noise |  | Mandatory Findings |

# DETERMINATION

On the basis of this initial evaluation:

|  | I find the proposed project, in accordance with those findings made pursuant to CEQA Guideline §15252, COULD NOT have a significant effect on the environment, and that an ENVIRONMENTAL ASSESSMENT with no significant impacts will be prepared. |
| --- | --- |
|  | I find that although the proposed project could have a significant effect on the environment, there will NOT be significant effects in this case because revisions in the project have been made by or agreed to by the project proponent. An ENVIRONMENTAL ASSESSMENT with no significant impacts will be prepared. |
|  | I find that the proposed project MAY have a significant effect(s) on the environment, and an ENVIRONMENTAL ASSESSMENT will be prepared. |
|  | I find that the proposed project MAY have a "potentially significant impact" on the environment, but at least one effect 1) has been adequately analyzed in an earlier document pursuant to applicable legal standards, and 2) has been addressed by mitigation measures based on the earlier analysis as described on attached sheets. An ENVIRONMENTAL ASSESSMENT is required, but it must analyze only the effects that remain to be addressed. |
|  | I find that although the proposed project could have a significant effect on the environment, because all potentially significant effects (a) have been analyzed adequately in an earlier ENVIRONMENTAL ASSESSMENT pursuant to applicable standards, and (b) have been avoided or mitigated pursuant to that earlier ENVIRONMENTAL ASSESSMENT, including revisions or mitigation measures that are imposed upon the proposed project, nothing further is required. |

Date    July 29, 2004	 	Signature: 			
			Steve Smith, Ph.D.	
			Program Supervisor

Planning, Rule Development & Area Sources

# ENVIRONMENTAL CHECKLIST AND DISCUSSION

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| I. AESTHETICS. Would the project: |  |  |  |
| - Have a substantial adverse effect on a scenic vista? |  |  |  |
| - Substantially damage scenic resources, including, but not limited to, trees, rock outcroppings, and historic buildings within a state scenic highway? |  |  |  |
| - Substantially degrade the existing visual character or quality of the site and its surroundings? |  |  |  |
| - Create a new source of substantial light or glare which would adversely affect day or nighttime views in the area? |  |  |  |

# Significance Criteria

The proposed project impacts on aesthetics will be considered significant if:

The project will block views from a scenic highway or corridor.

The project will adversely affect the visual continuity of the surrounding area.

The impacts on light and glare will be considered significant if the project adds lighting which would add glare to residential areas or sensitive receptors.

# Discussion

- a), b):  The continuation of using high VOC solvents in small batch-loaded cold cleaning and vapor degreasers will occur at existing aerospace and optics facilities located in industrial, institutional, or commercial areas.  Minor construction activities may be associated with the proposed amendments.  Construction is expected to consist of installation of off-the-shelf control technologies such as small afterburners or carbon absorbers so major construction equipment and material will not be needed and stockpiling of construction materials is not expected for the proposed project.

I. c). No new construction of buildings or other structures to install equipment is anticipated so scenic resources will not be obstructed and the existing visual character of any site in the vicinity of affected facilities will not be degraded.  New control equipment will be installed within the existing structure and will not change the existing visual character.

- d). There are no components in PAR 1122 that would require construction activities at night.  Further, degreasing activities typically occur inside industrial or commercial buildings.  Therefore, PAR 1122 is not expected to create a new source of substantial light or glare that would adversely affect day or nighttime views in the area.  Therefore, the proposed project is not expected to create significant adverse aesthetic impacts.

Based on the above consideration, significant adverse impacts to aesthetics are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| II. AGRICULTURE RESOURCES. Would the project: |  |  |  |
| a) Convert Prime Farmland, Unique Farmland, or Farmland of Statewide Importance (Farmland), as shown on the maps prepared pursuant to the Farmland mapping and Monitoring Program of the California Resources Agency, to non- agricultural use? |  |  |  |
| b) Conflict with existing zoning for agricultural use, or a Williamson Act contract? |  |  |  |
| c) Involve other changes in the existing environment which, due to their location or nature, could result in conversion of Farmland, to non-agricultural use? |  |  |  |

Significance Criteria

Project-related impacts on agricultural resources will be considered significant if any of the following conditions are met:

The proposed project conflicts with existing zoning or agricultural use or Williamson Act contracts.

The proposed project will convert prime farmland, unique farmland or farmland of statewide importance as shown on the maps prepared pursuant to the farmland mapping and monitoring program of the California Resources Agency, to non-agricultural use.

The proposed project would involve changes in the existing environment, which due to their location or nature, could result in conversion of farmland to non-agricultural uses.

# Discussion

- a) - c):  As previously discussed, minor construction may be associated with extending the exemption for small batch-loaded cold cleaning and vapor degreasers.  Further, degreasing activities subject to PAR 1122 would occur at existing aerospace and optics facilities located in industrial, institutional, or commercial areas.  Therefore, the proposed project would not result in any construction of new buildings or other structures that would convert farmland to non-agricultural use or conflict with zoning for agricultural use or a Williamson Act contract.  Since the proposed project would allow existing small batch-loaded cold cleaning and vapor degreasing operations to continue using high VOC solvents as long as specific conditions are met, there are no provisions in the proposed amended rule that would affect land use plans, policies, or regulations.  Land use and other planning considerations are determined by local governments and no land use or planning requirements will be altered by the proposed project.

Based on the above consideration, significant adverse impacts to agriculture resources are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | #### No Impact |
| --- | --- | --- | --- |
| III. AIR QUALITY. Would the project: |  |  |  |
| a) Conflict with or obstruct implementation of the applicable air quality plan? |  |  |  |
| b) Violate any air quality standard or contribute to an existing or projected air quality violation? |  |  |  |
| c) Result in a cumulatively considerable net increase of any criteria pollutant for which the project region is non-attainment under an applicable federal or state ambient air quality standard (including releasing emissions that exceed quantitative thresholds for ozone precursors)? |  |  |  |
| d) Expose sensitive receptors to substantial pollutant concentrations? |  |  |  |
| e) Create objectionable odors affecting a substantial number of people? |  |  |  |
| f) Diminish an existing air quality rule or future compliance requirement resulting in a significant increase in air pollutant(s)? |  |  |  |

Significance Criteria

Impacts will be evaluated and compared to the significance criteria in Table 2-2.  If impacts equal or exceed any of the criteria in Table 2-2, they will be considered significant.

# Discussion

III. a): PAR 1122 would not conflict with or obstruct, air quality plan implementation.  The primary purpose of the SCAQMD’s AQMP is to control emissions to attain and maintain all federal and state ambient air quality standards for the district.  The 2003 AQMP concluded that major reductions in emissions of VOC

TABLE  2-1

Air Quality Significance Thresholds

| ###### Mass Daily Thresholds |  |  |
| --- | --- | --- |
| Pollutant | Construction | Operation |
| NOx | 100 lbs/day | 55 lbs/day |
| VOC | 75 lbs/day | 55 lbs/day |
| PM10 | 150 lbs/day | 150 lbs/day |
| SOx | 150 lbs/day | 150 lbs/day |
| CO | 550 lbs/day | 550 lbs/day |
| Lead | 3 lbs/day | 3 lbs/day |
| ##### TAC, AHM, and Odor Thresholds |  |  |
| Toxic Air Contaminants (TACs) | Maximum Incremental Cancer Risk > 10 in 1 million Hazard Index > 1.0 (project increment) Hazard Index > 3.0 (facility-wide) |  |
| Odor | Project creates an odor nuisance pursuant to SCAQMD Rule 402 |  |
| ##### Ambient Air Quality for Criteria Pollutants |  |  |
| NO2 1-hour average annual average | 20 ug/m3 (= 1.0 pphm) 1 ug/m3 (= 0.05 pphm) |  |
| PM10 24-hour annual geometric mean 24-hour construction | 2.5 ug/m3 1.0 ug/m3 10.4 ug/m3 |  |
| Sulfate 24-hour average | 1 ug/m3 |  |
| CO 1-hour average 8-hour average | 1.1 mg/m3 (= 1.0 ppm) 0.50 mg/m3 (= 0.45 ppm) |  |

PM10 = particulate matter less than 10 microns in size, ug/m3 = microgram per cubic meter;  pphm = parts per hundred million;  mg/m3 = milligram per cubic meter;  ppm = parts per million; TAC = toxic air contaminant; AHM = Acutely Hazardous Material. NO2 = Nitrogen Oxide, CO = Carbon Monoxide, VOC = Volatile Organic Compounds, SOx = Sulfur Oxide.

and NOx are necessary to attain the air quality standards for ozone and PM10.  While VOC emission reductions will be foregone, the emissions are not significant.  As outlined in detail under III (f) below, if operators of affected facilities choose to install control equipment pursuant to proposed exemption in subparagraph (k)(1)(D), 17.1 pounds of VOC emissions per day are anticipated to be reduced instead of the emission reductions of 25 pounds per day that were anticipated had the exemption expired on January 1, 2005.  In addition, the 1997 amendments reduced VOC emissions by 40.2 tons per day by full implementation in 2010, so foregone emissions of eight nine pounds of VOC per day (0.0045 tons per day) will not prevent the SCAQMD’s progress in attaining the ambient air quality standards for ozone.

III. b), d), e):  There are two types of air quality effects resulting from implementing PAR 1122.  The first effect is emission reductions foregone as a result of permanently exempting specific types of equipment.  This effect is not an emission increase but a continuation of existing emissions and is discussed in more detail in item III. f).

The second effect is potential emission increases resulting from installing control equipment (construction emissions) and secondary emissions from operating pollution control equipment (operational emissions).  These air quality impacts are discussed in the following subsections.

Construction Emissions

Activities during construction that could potentially adversely affect air quality are those activities associated with the installation of the control equipment.  As noted previously, the two most likely alternative emission control systems expected to be used are a thermal oxidizer (afterburner), either a new one or tied into an existing one, or ductless fume hoods equipped with carbon filters to control the small degreasing activity at the affected facilities.  The construction-related activities attributable to facilities that would be installing control equipment would consist predominantly of cutting, welding, etc., as well as the construction worker needed to install the equipment.  For some ductless fume hoods, the design option can be mobile which would require no installation or ductwork.  These construction activities would not involve large-scale grading, slab pouring, or paving activities since the degreasing equipment being control is small and located in existing established facilities.  It is assumed that one construction worker is needed to deliver and install the equipment.  Therefore, on-road vehicle emissions are expected from the delivery truck that the construction worker will be driving.

Of all 13 affected facilities listed in Table 1-1, only one facility is expected to qualify for the permanent exemption based on cleaning electronic parts that are designed to travel over 100 miles above the earth’s surface.  The remaining 12 facilities could qualify for the permanent exemption if control equipment is installed before by the January June 1, 2005 compliance deadline.  Because of the expeditious compliance schedule, the “worst-case” scenario is that the delivery and installation of all 12 pieces of control equipment will take place on the same day.  Although simultaneous installation of control equipment is unlikely, it is possible since operators of affected facilities will only have three months between anticipated rule adoption and the compliance date to install the control device.  Table 2-1 summarizes the emissions from installing the 12 pieces of control equipment on the same day.  As shown in Table 2-1, the emissions to deliver and install the equipment would be less than the SCAQMD’s significance threshold, thus the potential adverse impact would be not significant.

Table 2-2

Construction Emissions from Installing Control Equipment

|  | CO | VOC | NOx | SOx | PM10 |
| --- | --- | --- | --- | --- | --- |
| EMFAC 2002 On-Road Delivery Truck Motor Vehicle Emission Factor for Year 2004 (pounds per mile) (1) | 0.02309 | 0.003148 | 0.029607 | 0.000243 | 0.000519 |
| Emissions from 12 worker vehicles (pounds per day) (2) | 6.9 | 0.94 | 8.9 | 0.07 | 0.16 |
| Air compressor (<50 HP) and Welder (<50 HP) Emission Factor (pounds per BHP-hr) (3) | 0.011 | 0.002 | 0.018 | 0.002 | 0.001 |
| Emissions from compressor and welder (pounds per day) (4) | 7.8 | 1.4 | 12.7 | 1.4 | 0.70 |
| Generator set (<50HP) Emission Factor (pounds per BHP-hr) (3) | 0.011 | 0.002 | 0.018 | 0.002 | 0.002 |
| Emissions from installation equipment (pounds per day) (5) | 3.9 | 0.72 | 6.5 | 0.72 | 0.72 |
| TOTAL CONSTRUCTION EMISSIONS – 12 FACILITIES (pounds per day) | 18.6 | 3.1 | 28.1 | 2.2 | 1.6 |
| SCAQMD CONSTRUCTION SIGNIFICANCE THRESHOLD (pounds per day) | 550 | 75 | 100 | 150 | 150 |
| SIGNIFICANT? | No | No | No | No | No |

(1) From CARB’s EMFAC 2002 (version 2.2) Burden Model (year 2005).  Weighted average delivery trucks (>8500 pounds)

(2)Assumes the construction worker in a delivery truck would travel 25 miles roundtrip since the affected facilities tend to be located in urban areas where industrial/commercial equipment is available locally.  Equation:  EF x 25 miles traveled x 12 workers per day = pounds per day

(3) SCAQMD CEQA Air Quality Handbook (April 1993), Table A9-8-A (diesel)

(4) Assumes 4 hours per day per piece of construction equipment (according to Richardson Engineering Services, 1996).  4 hours is used because during a normal 8 hr work day, there will be down time due to setup and breakdown.  Equation:  [EF x 1 compressor x 9 HP x 56 percent load factor x 12 facilities x 4 hours per day] +  [EF x 1 welder x 19 HP x 51 percent load factor x 12 facilities x 4 hours per day]= pounds per day

(5) Equation:  EF x 1 generator x 11 HP x 68 percent load factor x 12 facilities x 4 hours per day = pounds per day

Operational Emissions

Thermal Oxidizers

To estimate criteria pollutant emissions from thermal oxidizers, the SCAQMD used general default emission factors.  Currently, SCAQMD permitting staff requires that thermal oxidizers less than two million British thermal units (MMbtu) per hour to comply with a NOx concentration of 30 parts per million as BACT.  This translates to an emission factor of 36 pounds per million cubic feet (MMcf) of natural gas used as the combustion fuel.  The actual emission factors were derived from the Annual Emissions Reporting (AER) default emission factor of 130 pounds per MMcf (SCAQMD AER Program).  For CO, VOC, PM10, and SOx, the SCAQMD permitting staff uses the general AER default emission factors for all sizes of thermal oxidizers.

Twelve facilities were identified as having the potential to use thermal oxidizers as a means of complying with the conditions to qualify for the permanent exemption.  To calculate daily emissions, the number of affected facilities is multiplied by the assumed operating schedule and the amount of natural gas consumed, and then divided by the heating value of natural gas.  The result is multiplied by the criteria pollutant emission factor to determine the pounds per day of emissions.  At 10,000 cfm, the amount of natural gas consumed by a thermal oxidizer is 0.488 MMBTU per hour.  The heating value of natural gas is 1050 MMBTU/MMcf.

(12 facilities x 8 hrs/day x 0.488 MMBTU/hr)/(1050 MMBTU/MMcf) = 0.044 MMcf/day

Table 2-3 shows total criteria pollutant emissions generated by the facilities anticipated to install thermal oxidizers to reduce TAC emissions.  Table 4-8 shows that secondary criteria pollutant emissions from thermal oxidizers would not exceed the SCAQMD’s significance thresholds.

No additional workers are necessary to operate the control equipment.

Table 2-3

Estimated Operational Emissions from Thermal Oxidizers

| Criteria Pollutant | Emission Factor (pounds/MMcf) | MMcf/day | Total Emissions (pounds/day) | SCAQMD Operational Significance Threshold (pounds/day) | Significant? |
| --- | --- | --- | --- | --- | --- |
| NOx | 130 | 0.044 | 5.72 | 55 | No |
| VOC | 7 | 0.044 | 0.31 | 55 | No |
| CO | 35 | 0.044 | 1.54 | 550 | No |
| PM10 | 7.5 | 0.044 | 0.33 | 150 | No |
| SOx | 0.83 | 0.044 | 0.04 | 150 | No |

Ductless Fume Hoods

No operational emissions are expected from ductless fume hoods except the exhaust emissions from the truck transporting the worker who will periodically remove for the occasional replacement of the carbon filters.  Since it is unlikely the carbon filter replacement would not take place on the same day for the 12 facilities, the daily criteria pollutant emissions from the truck trip would be less than one pound per day. No additional workers are necessary to operate the control equipment

Air quality standards are not currently being violated because these affected facilities are exempt from the requirements of Rule 1122.  The proposed project will not induce violation of any air quality standards because affected facilities will simply continue being exempt from the requirements of Rule 1122 for small batch-loaded cold cleaners and vapor degreasers (see discussion III. f.) or generate small increases in emissions during construction and operation.  The same holds true in the exposure of sensitive receptors to VOC emissions from the exempt activity and creation of objectionable odors.  The operation of existing small batch-loaded cold cleaning and vapor degreasers is not expected to change at the affected facilities.  Since emission increases from the proposed project are minor and do not exceed any applicable significance thresholds, sensitive receptors are not expected to be exposed to substantial pollutant concentrations.  In fact, the conditions may improve over time as some facilities install control equipment and more facilities switch to low-VOC materials, such as water-based solvents.  In addition, local governments typically have ordinances that are intended to protect the public from adverse odors.  SCAQMD Rule 402 – Nuisance, also protects the public from adverse odor impacts.

III. c):  The potential adverse air quality impact related to implementation of PAR 1122 is a result of the foregone VOC emission reductions from permanently allowing the exemption from the requirements of Rule 1122 for small batch-loaded cold cleaners and vapor degreasers.  The daily foregone VOC emission reductions are less than the SCAQMD’s significance threshold and, therefore, are not considered to be cumulatively considerable as defined by CEQA Guidelines §15065(c).  Since PAR 1122 will not result in project-specific significant air quality impact, it is not expected to cause cumulative impacts in conjunction with other projects that may occur concurrently with or subsequent to the proposed project, the proposed project’s contribution to a potentially significant cumulative impact is rendered less than cumulatively considerable and, thus, is not significant (CEQA Guidelines §15064(i)(2)).

III. f):  As described in Chapter 1, affected facilities are generally not able to operate small batch-loaded cold cleaning and vapor degreasers in compliance with the requirements of Rule 1122.  Proposed amended Rule 1122 would permanently exempt batch-loaded cold cleaners and vapor degreasers used only for cleaning electronic parts that are designed to travel over 100 miles above the earth's surface, which is expected to result in a minimal amount of emission reductions permanently foregone.  No increase in VOC emissions from the affected equipment is anticipated and affected equipment will continue to be limited to 22 pounds per month per facility.  Equipment used solely for research and development programs, or laboratory tests in quality assurance laboratories would be permanently exempted from PAR 1122 resulting in a minimal amount of emission reductions foregone from affected facilities of one pound per day.  The proposed project also maintains the exemption for some types of small batch-loaded cold cleaning degreasers (e.g., those used for high precision optics, electrical or electronic components; etc.) if control equipment is installed.

Starting January 1, 2004, the exemption for the cleaning of photocurable resins from stereolithography equipment and models expired, and the rule now requires the use of cleaning materials with a VOC content of no more than 25 grams per liter of material for such cleaning.  Acetone, a VOC-exempt compound, is being used successfully to clean general purpose photocurable resins.  However, a certain class of stereolithography resins is not compatible with acetone and cannot be cleaned with this material.  Aqueous materials and other VOC-exempt compounds have been tested but did not effectively remove the residual resins.  A manufacturer of stereolithography equipment and resins has petitioned the SCAQMD to extend the rule exemption for the cleaning of stereolithography resins to allow additional time for research and testing of alternative cleaners.  The VOC emissions from the cleaning of stereolithography resins in the Basin are small (28 pounds per month or 1.3 pounds per day based on 22 working days per month).

As presented in Table 1-2, the total average daily VOC emissions from the operation of small batch-loaded cold cleaning and vapor degreasers in the district that are currently exempt from the requirements of Rule 1122 are 25 pounds per day.  By maintaining the exemptions for certain applications, eight nine pounds of VOC emissions per day originally anticipated for the rule would be foregone.  Table 2-4 outlines the equipment type eligible for continued use of the exemption and the amount of VOC emissions foregone as a result of maintaining the exemption.  The calculations assumed a “worst-case” scenario that all twelve eligible facilities will continue to take advantage of the exemption and install control equipment as required.  The minimum overall efficiency of the control equipment is 85.5 percent.  The emissions foregone takes into account the emission reduction anticipated if in compliance with current rule requirements at 25 grams per liter.  The emissions generated from solvents used have an average VOC content of 760 grams per liter.  Further, emission reductions anticipated from the rule requirements (96.7 percent) for electronic parts (one facility), and research equipment, are assumed to be permanently foregone emission reductions.  Since the exemption of photocurable resins from stereolithography equipment expires in 2008, the foregone emissions will not be permanent and are, thus, a delay of anticipated emission reductions.

The allowance to merge the operational VOC emission limit for consolidated facilities will not change the overall emissions from the individual facilities since the total emissions allowed do not exceed each individual facility limit (i.e., two facilities at a limit of 22 pounds per month each, if consolidated, are allowed 44 pounds per month overall).  This does not alter the conclusions in the Draft EA or worsen the environmental impact from the proposed project since the regional air quality impact will not change.

Table 2-4

Anticipated Foregone VOC Emission Reductions

| Equipment Type | Emissions from Affected Facilities (pounds per day) | Percent of Emission Reduction if Complying with Rule | Percent Efficiency of Control Equipment | VOC Emission Reduction Foregone (pounds per day) |
| --- | --- | --- | --- | --- |
| High-precision optics, electrical or electronic components; or aerospace and military applications for cleaning solar cells, laser hardware, fluid systems, and space vehicle components | 20 | 96.7 | 85.5 | 2.2 a |
| Electronic parts that are designed to travel over 100 miles above the earth's surface | 5 | 96.7 | n/a | 4.8 b |
| Equipment used solely for research and development programs, or laboratory tests in quality assurance laboratories | 1 | 96.7 | n/a | 0.97 b |
| Equipment used for cleaning of photocurable resins from stereolithography equipment and models | 1.3 |  | n/a | 1.2 |
| TOTAL FOREGONE VOC EMISSION REDUCTIONS (pounds per day) |  |  |  | 9.2 8 |

Sample Equation: a [emissions (#/day) x (1- percent control)] – [emissions x (1- percent from rule compliance)] = foregone emission reductions (#/day)

b [emissions x (1- percent from rule compliance)] = foregone emission reductions (#/day)

The foregone emission reductions along with the operational emissions from the control equipment (<one pound VOC per day) are less than the SCAQMD significance threshold for VOC, which is 55 pounds per day.  While there is an adverse air quality impact as a result of the proposed amendments, the air quality impact is not significant.  As noted in Tables 2-2 and 2-3, the criteria pollutants do not exceed the significance thresholds during either the construction or the operational phases of the project and therefore, significant adverse impacts to air quality are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| IV. BIOLOGICAL RESOURCES. Would the project: |  |  |  |
| a) Have a substantial adverse effect, either directly or through habitat modifications, on any species identified as a candidate, sensitive, or special status species in local or regional plans, policies, or regulations, or by the California Department of Fish and Game or U.S. Fish and Wildlife Service? |  |  |  |
| b) Have a substantial adverse effect on any riparian habitat or other sensitive natural community identified in local or regional plans, policies, or regulations, or by the California Department of Fish and Game or U.S. Fish and Wildlife Service? |  |  |  |
| c) Have a substantial adverse effect on federally protected wetlands as defined by §404 of the Clean Water Act (including, but not limited to, marsh, vernal pool, coastal, etc.) through direct removal, filling, hydrological interruption, or other means? |  |  |  |
| d) Interfere substantially with the movement of any native resident or migratory fish or wildlife species or with established native resident or migratory wildlife corridors, or impede the use of native wildlife nursery sites? |  |  |  |
| e) Conflicting with any local policies or ordinances protecting biological resources, such as a tree preservation policy or ordinance? |  |  |  |
| f) Conflict with the provisions of an adopted Habitat Conservation plan, Natural Community Conservation Plan, or other approved local, regional, or state habitat conservation plan? |  |  |  |

Significance Criteria

Impacts on biological resources will be considered significant if any of the following criteria apply:

The project results in a loss of plant communities or animal habitat considered to be rare, threatened or endangered by federal, state or local agencies.

The project interferes substantially with the movement of any resident or migratory wildlife species.

The project adversely affects aquatic communities through construction or operation of the project.

# Discussion

IV. a), b), d): The proposed amendments would not increase VOC emissions from affected equipment, simply maintain existing emissions at existing facilities.  Slight emission increases may occur at those affected facilities where operators choose to install control equipment.  Emissions associated with installing control equipment will not exceed any applicable significance thresholds, either for construction or operation.  Therefore, the proposed amendments to Rule 1122 will have no direct or indirect impacts that could adversely affect plant or animal species or the habitats on which they rely in the SCAQMD’s jurisdiction.  The exempt activity takes place at existing facilities and, eventually, the net effect of implementing the proposed amended rule will be improved air quality resulting from reduced VOC emissions, which is expected to be beneficial for both plant and animal life.  Modifications at existing affected facilities to switch to low-VOC materials, such as water-based, or alternative solvent degreasing activities would not require acquisition of additional land or further conversions of riparian habitats or sensitive natural communities where endangered or sensitive species may be found.

IV. c): Acquisition of protected wetlands is not expected to be necessary to continue the exemption or to eventually switch to low-VOC materials, such as water-based, or alternative solvent degreasing activities.  Affected facilities would be allowed to continue existing operating practices so the proposed amended rule will not directly remove, fill or interrupt any hydrological system or have an adverse effect on federally protected wetlands.

- e), f):There are no provisions in the proposed amended rule that would adversely affect land use plans, local policies or ordinances, or regulations.  Land use and other planning considerations are determined by local governments and no land use or planning requirements will be altered by the proposed project.  The proposed amended Rule 1122 would not affect in any way habitat conservation or natural community conservation plans, agricultural resources or operations, and would not create divisions in any existing communities.

Based on the above consideration, significant adverse impacts to biological resources are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| V. CULTURAL RESOURCES. Would the project: |  |  |  |
| - Cause a substantial adverse change in the significance of a historical resource as defined in §15064.5? |  |  |  |
| - Cause a substantial adverse change in the significance of an archaeological resource as defined in §15064.5? |  |  |  |
| - Directly or indirectly destroy a unique paleontological resource or site or unique geologic feature? |  |  |  |
| d) Disturb any human remains, including those interred outside a formal cemeteries? |  |  |  |

Significance Criteria

Impacts to cultural resources will be considered significant if:

The project results in the disturbance of a significant prehistoric or historic archaeological site or a property of historic or cultural significance to a community or ethnic or social group.

Unique paleontological resources are present that could be disturbed by construction of the proposed project.

The project would disturb human remains.

# Discussion

V. a) - d): There are existing laws in place that are designed to protect and mitigate potential impacts to cultural resources.  Degreasing operations currently exempt from Rule 1122 take place at existing aerospace and optics facilities and won’t require construction activities such as grading, trenching, etc.  Therefore, cultural resources would not be disturbed.  As a result, the proposed project has no potential to cause a substantial adverse change to a historical or archaeological resource, directly or indirectly destroy a unique paleontological resource or site or unique geologic feature, or disturb any human remains, including those interred outside a formal cemeteries.

Based on the above consideration, significant adverse impacts to cultural resources are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| VI. ENERGY. Would the project: |  |  |  |
| a) Conflict with adopted energy conservation plans? |  |  |  |
| b) Result in the need for new or substantially altered power or natural gas utility systems? |  |  |  |
| c) Create any significant effects on local or regional energy supplies and on requirements for additional energy? |  |  |  |
| d) Create any significant effects on peak and base period demands for electricity and other forms of energy? |  |  |  |
| e) Comply with existing energy standards? |  |  |  |

Significance Criteria

Impacts to energy and mineral resources will be considered significant if any of the following criteria are met:

The project conflicts with adopted energy conservation plans or standards.

The project results in substantial depletion of existing energy resource supplies.

An increase in demand for utilities impacts the current capacities of the electric and natural gas utilities.

The project uses non-renewable resources in a wasteful and/or inefficient manner.

# Discussion

VI. a), e): Maintaining the existing exemption at affected facilities will not conflict with adopted energy conservation plans or cause any affected facility to be out of compliance with existing energy standards because affected equipment would be allowed to continue existing operations.  PAR 1122 will not increase the demand for electricity at affected facilities, and would not be expected to interfere with existing or future energy conservation plans because these are typically targeted to residential consumers, etc.

VI. b), c), d): The operation of existing small batch-loaded cold cleaning and vapor degreasers is not expected to change at the affected facilities.  However, approximately 12 facilities could use a thermal oxidizer to qualify for a permanent exemption for small degreasers.  To estimate natural gas fuel usage from thermal oxidizer operation, the SCAQMD assumed that the 12 units (one unit per facility) would operate eight hours per day, six days per week, 52 weeks per year and fire natural gas only.  At an exhaust emission flow rate of 10,000 cfm, the amount of natural gas consumed is 0.488 MMBTU/hr and 28 kW of instantaneous power.

(12 facilities x 8 hrs/day x 6 days/wk x 52 wks/yr x 0.488 MMBTU/hr)/(1050 MMBTU/MMcf) = 13.9 MMcf per year

Table 2-5 lists the projected natural gas impacts associated with the operational phase of the proposed amendments.  The natural gas usage from the proposed project is negligible to the remaining capacity of natural gas available in the district.

TABLE 2-5

Total Projected Natural Gas Usage from Thermal Oxidizer Operations

| Year | Projected Natural Gas Supplya (mmcf/yr) | Total Natural Gas Usage (mmcf/yr) | Total Impact % on Natural Gas Supplies | Significant? |
| --- | --- | --- | --- | --- |
| 2010 | 2,657,000 | 13.9 | 0.0005 | No |

a From Table 3.3-6 of the 2003 AQMP Final EIR from California Energy Commission (2002)

Electrical energy impacts (estimated to be approximately 336 kW = 12  facilities x 28 kW at 10,000 cfm) associated with ancillary equipment (e.g., fans, motors, etc.) used in conjunction with the thermal oxidizers will not constitute significant adverse energy impacts.  Furthermore, the small amount of additional fuel that may be used to generate electricity would be negligible compared to existing supplies and, thus, would not substantially deplete existing energy resources.

Therefore, based on the foregoing analysis, the SCAQMD has determined that operational-related activities associated with the implementation of the proposed amendments is necessary and will not use energy in a wasteful manner: will not result in substantial depletion of existing energy resource supplies; nor will significant amounts of fuel be needed when compared to existing supplies.  Furthermore, if additional fuel is needed to generate electricity for electric fans or motors used in conjunction with thermal oxidizers at affected facilities, it would not be a wasteful use of energy nor substantially deplete existing energy resources.

Based on the above considerations, significant adverse impacts to energy are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| VII. GEOLOGY AND SOILS. Would the project: |  |  |  |
| a) Expose people or structures to potential substantial adverse effects, including the risk of loss, injury, or death involving: |  |  |  |
| - Rupture of a known earthquake fault, as delineated on the most recent Alquist-Priolo Earthquake Fault Zoning Map issued by the State Geologist for the area or based on other substantial evidence of a known fault? |  |  |  |
| - Strong seismic ground shaking? |  |  |  |
| - Seismic–related ground failure, including liquefaction? |  |  |  |
| - Landslides? |  |  |  |
| b) Result in substantial soil erosion or the loss of topsoil? |  |  |  |
| c) Be located on a geologic unit or soil that is unstable or that would become unstable as a result of the project, and potentially result in on- or off-site landslide, lateral spreading, subsidence, liquefaction or collapse? |  |  |  |
| d) Be located on expansive soil, as defined in Table 18-1-B of the Uniform Building Code (1994), creating substantial risks to life or property? |  |  |  |
| e) Have soils incapable of adequately supporting the use of septic tanks or alternative waste water disposal systems where sewers are not available for the disposal of waste water? |  |  |  |

Significance Criteria

Impacts on the geological environment will be considered significant if any of the following criteria apply:

Topographic alterations would result in significant changes, disruptions, displacement, excavation, compaction or over covering of large amounts of soil.

Unique geological resources (paleontological resources or unique outcrops) are present that could be disturbed by the construction of the proposed project.

Exposure of people or structures to major geologic hazards such as earthquake surface rupture, ground shaking, liquefaction or landslides.

Secondary seismic effects could occur which could damage facility structures, e.g., liquefaction.

Other geological hazards exist which could adversely affect the facility, e.g., landslides, mudslides.

# Discussion

VII. a): The exempt degreasing activity takes place at existing affected facilities so the proposed amendments to Rule 1122 will not expose people to potential substantial geological effects greater than what they are exposed to already.  Maintaining the existing exemption for existing small batch-loaded cold cleaners and vapor degreasers at aerospace and optics facilities will not expose people or structures to risks of loss, injury, or death involving: rupture of an earthquake fault, seismic ground shaking, ground failure or landslides.

VII. b): The proposed project will not require construction activities (e.g., grading, trenching, refilling and repaving), so there are no potential impacts to existing geophysical conditions.  Because the exempt degreasing activity takes place at existing facilities on established foundations, no soil is expected to be disrupted.  Therefore, no substantial soil erosion or loss of topsoil is expected from the existing affected facilities as a result of delaying the future compliance requirement for small batch-loaded cold cleaners and vapor degreasers.

VII. c), d):  The exempt activity takes place at existing affected facilities and, therefore, will not involve locating any structures on soil that is unstable or expansive.  However, as already noted, no soil disturbance is anticipated from delaying the compliance requirement, therefore, no further destabilization of unstable soils would be expected that could cause on- or off-site landslides, lateral spreading, subsidence, liquefaction or collapse.

VII. e):  The proposed project does not involve the installation of septic tanks or alternative waste water disposal systems.  Therefore, this type of soil impact will not occur.

Based on the above considerations, significant adverse impacts to geology and soils are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| VIII. HAZARDS AND HAZARDOUS MATERIALS. Would the project: |  |  |  |
| a) Create a significant hazard to the public or the environment through the routine transport, use, and disposal of hazardous materials? |  |  |  |
| - Create a significant hazard to the public or the environment through reasonably foreseeable upset and accident conditions involving the release of hazardous materials into the environment? |  |  |  |
| - Emit hazardous emissions, or handle hazardous or acutely hazardous materials, substances, or waste within one-quarter mile of an existing or proposed school? |  |  |  |
| - Be located on a site which is included on a list of hazardous materials sites compiled pursuant to Government Code §65962.5 and, as a result, would create a significant hazard to the public or the environment? |  |  |  |
| - For a project located within an airport land use plan or, where such a plan has not been adopted, within two miles of a public airport or public use airport, would the project result in a safety hazard for people residing or working in the project area? |  |  |  |
| - For a project within the vicinity of a private airstrip, would the project result in a safety hazard for people residing or working in the project area? |  |  |  |
| - Impair implementation of or physically interfere with an adopted emergency response plan or emergency evacuation plan? |  |  |  |
| - Expose people or structures to a significant risk of loss, injury or death involving wildland fires, including where wildlands are adjacent to urbanized areas or where residences are intermixed with wildlands? |  |  |  |
| - Significantly increased fire hazard in areas with flammable materials? |  |  |  |

Significance Criteria

Impacts associated with hazards will be considered significant if any of the following occur:

Non-compliance with any applicable design code or regulation.

Non-conformance to National Fire Protection Association standards.

Non-conformance to regulations or generally accepted industry practices related to operating policy and procedures concerning the design, construction, security, leak detection, spill containment or fire protection.

Exposure to hazardous chemicals in concentrations equal to or greater than the Emergency Response Planning Guideline (ERPG) 2 levels.

# Discussion

VIII. a), b), c): The operation of existing small batch-loaded cold cleaning and vapor degreasers is not expected to change at the affected facilities and, thus, the amount of solvents used in the exempt activity is not expected to change.  While the proposed amended rule allows the continued use of high VOC solvents for small batch-loaded cold cleaners and vapor degreasers, no additional transport of the solvents is expected and, thus, no new hazards to the public will be created through transport, use or disposal of hazardous materials.  Consequently, the proposed amendments to Rule 1122 will not create a significant new hazard to the public or create a reasonably foreseeable upset involving the release of hazardous materials.  Similarly, emissions from affected facilities will not increase except in situations where operators install control equipment.  As noted in the air quality discussion, potential emission increases do not exceed any applicable significance thresholds.

VIII. d):  Government code §65962.5 refers to hazardous waste handling practices at facilities subject to the Resources Conservation and Recovery Act (RCRA).  Since the proposed project would allow a continuation of existing degreasing operations from small batch-loaded cold cleaners and vapor degreasers, hazardous waste handling practices, if any, at regulated facilities would not be affected.

VIII. e), f):  Even for facilities that may be located near public airports or private airstrips, the proposed project will not create new safety hazards because the operation of the small batch-loaded cold cleaners and vapor degreasers is not expected to change.  Operators at affected facilities may install control equipment, but this is expected to occur inside existing structures, so no hazards would be created near any airports or airstrips.

VIII. g):  The exempt degreasing activity has no effect on a facility’s ability to comply, and is not expected to interfere, with all applicable rules and regulations, including any government codes, airport land use plans, adopted emergency response plans and emergency evacuation plans.

VIII. h,) i):  Affected facilities currently use alcohols, such as IPA, in their solvent degreasing process.  The proposed project will not alter existing solvent usage.  IPA is flammable but fire suppression systems to control any potential fire hazards are already in place.  Some facilities may eventually comply with lower VOC content limit requirements, which is likely to happen through reformulation of the solvent or conversion to alternative technologies.  It is anticipated that the reformulation will entail the use of water-based components or low-VOC materials less hazardous or flammable than currently being used.

The Uniform Fire Code and Uniform Building Code set standards intended to minimize risks from flammable or otherwise hazardous materials.  Local jurisdictions are required to adopt the uniform codes or comparable regulations.  Local fire agencies require permits for the use or storage of hazardous materials and permit modifications for proposed increases in their use.  Permit conditions depend on the type and quantity of the hazardous materials at the facility.  Permit conditions may include, but are not limited to, specifications for sprinkler systems, electrical systems, ventilation, and containment.  The fire departments make annual business inspections to ensure compliance with permit conditions and other appropriate regulations.  Consequently, local fire departments ensure that adequate permit conditions are in place to protect against potential risk of upset from the use of hazardous materials.  However, any use of hazardous materials at affected facilities is not expected to change as a result of implementing the proposed project.

Based on the above considerations, significant adverse hazards and hazardous materials impacts are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| IX. HYDROLOGY AND WATER QUALITY. Would the project: |  |  |  |
| a) Violate any water quality standards or waste discharge requirements? |  |  |  |
| - Substantially deplete groundwater supplies or interfere substantially with groundwater recharge such that there would be a net deficit in aquifer volume or a lowering of the local groundwater table level (e.g. the production rate of pre-existing nearby wells would drop to a level which would not support existing land uses or planned uses for which permits have been granted)? |  |  |  |
| c) Substantially alter the existing drainage pattern of the site or area, including through alteration of the course of a stream or river, in a manner that would result in substantial erosion or siltation on- or off-site? |  |  |  |
| d) Substantially alter the existing drainage pattern of the site or area, including through alteration of the course of a stream or river, or substantially increase the rate or amount of surface runoff in a manner that would result in flooding on- or off-site? |  |  |  |
| e) Create or contribute runoff water which would exceed the capacity of existing or planned stormwater drainage systems or provide substantial additional sources of polluted runoff? |  |  |  |
| f) Otherwise substantially degrade water quality? |  |  |  |
| g) Place housing within a 100-year flood hazard area as mapped on a federal Flood Hazard Boundary or Flood Insurance Rate Map or other flood hazard delineation map? |  |  |  |
| h) Place within a 100-year flood hazard area structures which would impede or redirect flood flaws? |  |  |  |
| i) Expose people or structures to a significant risk of loss, injury or death involving flooding, including flooding as a result of the failure of a levee or dam? |  |  |  |
| j) Inundation by seiche, tsunami, or mudflow? |  |  |  |
| k) Exceed wastewater treatment requirements of the applicable Regional Water Quality Control Board? |  |  |  |
| l) Require or result in the construction of new water or wastewater treatment facilities or expansion of existing facilities, the construction of which could cause significant environmental effects? |  |  |  |
| m) Require or result in the construction of new storm water drainage facilities or expansion of existing facilities, the construction of which could cause significant environmental effects? |  |  |  |
| n) Have sufficient water supplies available to serve the project from existing entitlements and resources, or are new or expanded entitlements needed? |  |  |  |
| o) Require in a determination by the wastewater treatment provider which serves or may serve the project that it has adequate capacity to serve the project's projected demand in addition to the provider's existing commitments? |  |  |  |

Significance Criteria

Potential impacts on water resources will be considered significant if any of the following criteria apply:

Water Quality:

The project will cause degradation or depletion of ground water resources substantially affecting current or future uses.

The project will cause the degradation of surface water substantially affecting current or future uses.

The project will result in a violation of National Pollutant Discharge Elimination System (NPDES) permit requirements.

The capacities of existing or proposed wastewater treatment facilities and the sanitary sewer system are not sufficient to meet the needs of the project.

The project results in substantial increases in the area of impervious surfaces, such that interference with groundwater recharge efforts occurs.

The project results in alterations to the course or flow of floodwaters.

Water Demand:

The existing water supply does not have the capacity to meet the increased demands of the project, or the project would use a substantial amount of potable water.

The project increases demand for water by more than five million gallons per day.

# Discussion

IX. a), b), f), n), o): Delaying a future compliance requirement at affected facilities will have no direct or indirect impact on hydrology and water quality because the solvent degreasers used in affected facilities do not typically require the use of water in any way.  Therefore, PAR 1122 will not adversely affect water resources, water quality standards, groundwater supplies, water quality degradation, existing water supplies or wastewater treatment facilities.

IX. g), h): PAR 1122 will not place new housing in a 100-year flood hazard areas.  Exempt degreasing activities take place at existing aerospace and optics facilities so any flood hazards would be part of the existing setting.

IX. c), d), e):  The proposed project would allow continued operation of small batch-loaded cold cleaning and vapor degreasers at existing facilities.  Consequently, no construction activities will be necessary to comply with PAR 1122, so the proposed project will not alter any existing drainage patterns, increase the rate or amount of surface runoff water that would exceed the capacity of existing or planned stormwater drainage systems.

IX. l), m): Because no water or waste results from the exempt activity, the proposed project would not generate additional volumes of waterwater that could exceed the capacity of existing stormwater drainage systems or require the construction of new wastewater or stormwater drainage facilities.

IX. k): Since the proposed project will not change degreasing operations at affected facilities, no changes to  existing wastewater treatment permits would be necessary so they would still be expected to comply with existing wastewater treatment requirements of the applicable Regional Water Quality Control Board.

IX. i), j):  Since PAR 1122 affects existing facilities and does not require construction of new facilities, it will not alter existing flood risks or risks from seiches, tsunami’s or mudflow conditions.  Minor construction activities could occur within existing affecting facilities, but construction is not expected to require construction of new facilities in areas that could be affected by floods, seiches, etc.

Based on the above considerations, significant adverse impacts to hydrology and water quality are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| X. LAND USE AND PLANNING. Would the project: |  |  |  |
| a) Physically divide an established community? |  |  |  |
| b) Conflict with any applicable land use plan, policy, or regulation of an agency with jurisdiction over the project (including, but not limited to the general plan, specific plan, local coastal program or zoning ordinance) adopted for the purpose of avoiding or mitigating an environmental effect? |  |  |  |
| c) Conflict with any applicable habitat conservation or natural community conservation plan? |  |  |  |

Significance Criteria

Land use and planning impacts will be considered significant if the project conflicts with the land use and zoning designations established by local jurisdictions.

# Discussion

X. a.): Delaying a future compliance requirement at affected facilities will not create divisions in any existing communities because this provision applies generally to operations at existing facilities.

X. b), c): Operations at aerospace and optics facilities would still be expected to comply, and not interfere, with any applicable land use plans, zoning ordinances, habitat conservation or natural community conservation plans.  There are no provisions of the proposed project that would directly affect these plans, policies, or regulations.  Land use and other planning considerations are determined by local governments and no present or planned land uses in the region or planning requirements will be altered by the proposed project.

Based on the above considerations, significant adverse impacts to land use and planning are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| XI. MINERAL RESOURCES. Would the project: |  |  |  |
| a) Result in the loss of availability of a known mineral resource that would be of value to the region and the residents of the state? |  |  |  |
| b) Result in the loss of availability of a locally-important mineral resource recovery site delineated on a local general plan, specific plan or other land use plan? |  |  |  |

Significance Criteria

Project-related impacts on mineral resources will be considered significant if any of the following conditions are met:

The project would result in the loss of availability of a known mineral resource that would be of value to the region and the residents of the state.

The proposed project results in the loss of availability of a locally-important mineral resource recovery site delineated on a local general plan, specific plan or other land use plan.

# Discussion

XI. a), b): There are no provisions of the proposed amended rule that would directly result in the loss of availability of a known mineral resource, such as aggregate, coal, shale, etc. of value to the region and the residents of the state, or of a locally-important mineral resource recovery site delineated on a local general plan, specific plan or other land use plan.  The proposed project would simply continue an exemption for small batch-loaded cold cleaners and vapor degreasers at existing facilities.

Based on the above considerations, significant adverse impacts to mineral resources are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| XII. NOISE. Would the project result in: |  |  |  |
| a) Exposure of persons to or generation of noise levels in excess of standards established in the local general plan or noise ordinance, or applicable standards of other agencies? |  |  |  |
| b) Exposure of persons to or generation of excessive groundborne vibration or groundborne noise levels? |  |  |  |
| c) A substantial permanent increase in ambient noise levels in the project vicinity above levels existing without the project? |  |  |  |
| d) A substantial temporary or periodic increase in ambient noise levels in the project vicinity above levels existing without the project? |  |  |  |
| e) For a project located within an airport land use plan or, where such a plan has not been adopted, within two miles of a public airport or public use airport, would the project expose people residing or working in the project area to excessive noise levels? |  |  |  |
| f) For a project within the vicinity of a private airship, would the project expose people residing or working in the project area to excessive noise levels? |  |  |  |

Significance Criteria

Impacts on noise will be considered significant if:

Construction noise levels exceed the local noise ordinances or, if the noise threshold is currently exceeded, project noise sources increase ambient noise levels by more than three decibels (dBA) at the site boundary.  Construction noise levels will be considered significant if they exceed federal Occupational Safety and Health Administration (OSHA) noise standards for workers.

The proposed project operational noise levels exceed any of the local noise ordinances at the site boundary or, if the noise threshold is currently exceeded, project noise sources increase ambient noise levels by more than three dBA at the site boundary.

# Discussion

XII. a), b), c), d): There is no generation of noise, excessive groundborne vibration, or substantial increase in ambient noise levels from the operation of small batch-loaded cold cleaners and vapor degreasers.  Thus, delaying a future compliance requirement regarding the VOC content of solvents used by small batch-loaded cold cleaners and vapor degreasers at affected facilities would have no additional noise impacts.

XII. e), f): Noise levels will either not change or will decline as a result of the proposed project and, therefore, will not have an adverse noise impact even if a facility is located near an airport or private airstrip.

Based on the above considerations, significant adverse impacts to noise are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| XIII. POPULATION AND HOUSING. Would the project: |  |  |  |
| a) Induce substantial growth in an area either directly (for example, by proposing new homes and businesses) or indirectly (e.g. through extension of roads or other infrastructure)? |  |  |  |
| b) Displace substantial numbers of existing housing, necessitating the construction of replacement housing elsewhere? |  |  |  |
| c) Displace substantial numbers of people, necessitating the construction of replacement housing elsewhere? |  |  |  |

Significance Criteria

Impacts of the proposed project on population and housing will be considered significant if the following criteria are exceeded:

The demand for temporary or permanent housing exceeds the existing supply.

The proposed project produces additional population, housing or employment inconsistent with adopted plans either in terms of overall amount or location.

# Discussion

XIII. a), b), c):  Human population in the SCAQMD’s jurisdiction is anticipated to grow regardless of implementing the proposed project.  The proposal would continue the exemption of small batch-loaded cold cleaners and vapor degreasers from the requirements of Rule 1122, which will not require additional employees since the proposed project would allow existing operations to continue.  The population will not grow directly as a result of the proposed amended rule and the exempt activity will not indirectly induce growth in the area of the aerospace or optic facilities.  The construction of single- or multiple-family housing units would not be required as a result of implementing the proposed project.  Therefore, existing housing or populations in the district are not anticipated to be displaced necessitating the construction of replacement housing elsewhere.

Based on the above considerations, significant adverse impacts to population and housing are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| XIV. PUBLIC SERVICES. Would the proposal result in substantial adverse physical impacts associated with the provision of new or physically altered governmental facilities, need for new or physically altered government facilities, the construction of which could cause significant environmental impacts, in order to maintain acceptable service ratios, response times or other performance objectives for any of the following public services: |  |  |  |
| a) Fire protection? |  |  |  |
| b) Police protection? |  |  |  |
| c) Schools? |  |  |  |
| d) Parks? |  |  |  |
| e) Other public facilities? |  |  |  |

Significance Criteria

Impacts on public services will be considered significant if the project results in substantial adverse physical impacts associated with the provision of new or physically altered governmental facilities, or the need for new or physically altered government facilities, the construction of which could cause significant environmental impacts, in order to maintain acceptable service ratios, response time or other performance objectives.

# Discussion

XIV. a), b): Although affected facilities could install afterburners to continue qualifying for the exemption for small batch-loaded cold cleaners and vapor degreasers, at most 12 facilities would install this type of control equipment.  Since operators of affected facilities who install control equipment would be required to comply with all relevant Uniform Fire Code standards, existing fire and police services are expected to be able to handle any fire emergencies should they occur.

XIV. c), d):  Because the operation of the small batch-loaded cold cleaners and vapor degreasers is not expected to change, affected facilities are not expected to require new employees.  Consequently, no new impacts to schools, parks or other recreational facilities are foreseen as a result of implementing the proposed amendments to Rule 1122.

XIV. e):  The proposal would not result in the need for new or physically altered government facilities in order to maintain acceptable service ratios, response times or other performance objectives because operators of affected facilities will be able to continue existing operations.

Based on the above considerations, significant adverse impacts to public services are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| XV. RECREATION. |  |  |  |
| a) Would the project increase the use of existing neighborhood and regional parks or other recreational facilities such that substantial physical deterioration of the facility would occur or be accelerated.? |  |  |  |
| b) Does the project include recreational facilities or require the construction or expansion of recreational facilities that might have an adverse physical effect on the environment? |  |  |  |

Significance Criteria

Impacts to recreation will be considered significant if:

The project results in an increased demand for neighborhood or regional parks or other recreational facilities.

The project adversely effects existing recreational opportunities.

# Discussion

XV. a), b): As discussed under “Land Use and Planning” above, there are no provisions in the proposed project that would affect land use plans, policies or ordinances, or regulations.  Land use and other planning considerations are determined by local governments; no land use or planning requirements will be altered by the proposal.  As already noted in item XII, Population and Housing, the proposed project is not expected to increase population growth in the district because no additional employees would be required at affected facilities so no additional demand for parks is anticipated.  Further, the proposed amendments would not increase the use of existing neighborhood and regional parks or other recreational facilities or include recreational facilities or require the construction or expansion of recreational facilities that might have an adverse physical effect on the environment.

Based on the above considerations, significant adverse impacts to recreation are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact |  | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- |
| XVI. SOLID/HAZARDOUS WASTE. Would the project: |  |  |  |  |
| a) Be served by a landfill with sufficient permitted capacity to accommodate the project’s solid waste disposal needs? |  |  |  |  |
| b) Comply with federal, state, and local statutes and regulations related to solid and hazardous waste? |  |  |  |  |

Significance Criteria

The proposed project impacts on solid/hazardous waste will be considered significant if the following occur:

The generation and disposal of hazardous and non-hazardous waste exceeds the capacity of designated landfills.

# Discussion

XVI. a), b): If a facility chooses to install a ductless fume hood equipped with a carbon filter, the carbon must be periodically removed and replaced.  As soon as the activated carbon becomes saturated, it is removed and sent to a reprocessing facility for reactivation.  In the reactivation process, the adsorbed contaminants are driven off at medium temperatures and later oxidized at higher temperature.  A supply of fresh carbon is installed in the system allowing it to return to normal operations.  Therefore, no solid or hazardous waste results from the carbon replacement process.  For thermal oxidizers applications, no solid or hazardous waste is produced from the control process.  It is expected that affected facilities would continue to comply with established federal, state, and local statutes and regulations related to solid and hazardous waste handling and disposal.  Therefore, potential solid waste impacts are considered not significant.

Based on the above considerations, significant adverse impacts to solid/hazardous waste are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| XVII. TRANSPORTATION/CIRCULATION Would the project: |  |  |  |
| a) Cause an increase in traffic which is substantial in relation to the existing traffic load and capacity of the street system (i.e., result in a substantial increase in either the number of vehicle trips, the volume to capacity ratio on roads, or congestion at intersections)? |  |  |  |
| - Exceed, either individually or cumulatively, a level of service standard established by the county congestion management agency for designated roads or highways? |  |  |  |
| - Result in a change in air traffic patterns, including either an increase in traffic levels or a change in location that results in substantial safety risks? |  |  |  |
| - Substantially increase hazards due to a design feature (e.g. sharp curves or dangerous intersections) or incompatible uses (e.g. farm equipment)? |  |  |  |
| e) Result in inadequate emergency access? |  |  |  |
| f) Result in inadequate parking capacity? |  |  |  |
| g) Conflict with adopted policies, plans, or programs supporting alternative transportation (e.g. bus turnouts, bicycle racks)? |  |  |  |

Significance Criteria

Impacts on transportation/traffic will be considered significant if any of the following criteria apply:

Peak period levels on major arterials are disrupted to a point where level of service (LOS) is reduced to D, E or F for more than one month.

An intersection’s volume to capacity ratio increase by 0.02 (two percent) or more when the LOS is already D, E or F.

A major roadway is closed to all through traffic, and no alternate route is available.

There is an increase in traffic that is substantial in relation to the existing traffic load and capacity of the street system.

The demand for parking facilities is substantially increased.

Water borne, rail car or air traffic is substantially altered.

Traffic hazards to motor vehicles, bicyclists or pedestrians are substantially increased.

The need for more than 350 employees

An increase in heavy-duty transport truck traffic to and/or from the facility by more than 350 truck round trips per day

Increase customer traffic by more than 700 visits per day.

# Discussion

XVII. a), b), f): The operation of the small batch-loaded cold cleaners and vapor degreasers is not expected to change and, therefore, no additional transportation/circulation impacts will directly or indirectly result from delaying the exemption of the small batch-loaded cold cleaners and vapor degreasers from the requirements of Rule 1122.  No new employees are needed to continue the operation of the exempt activity and, if applicable, the control equipment.  One occasional worker needed to replace the carbon filter on the ductless fume hood equipment is well below the transportation significance threshold of 350 truck round trips per day and, therefore, no new worker trips that could increase traffic or affect in any way the level of service designation for any roadways will result from the proposed amendments.  Similarly, additional parking would not be required from implementing PAR 1122.  Because the operation of the small batch-loaded cold cleaners and vapor degreasers is not expected to change, no new or additional raw materials will be needed and, therefore, no transport trips that could affect the level of service for roadways will be generated from the continued operation of the exempt activity.

XVII. c):  Air traffic patterns are not expected to be directly or indirectly affected by the proposed amended rule because the exempt activity will not require any air transportation nor will it interfere with air traffic.  All applicable local, state and federal requirements would continue to be complied with so no increase in any safety risks is expected.

XVII. d), e): The proposed amendments to Rule 1122 do not have direct or indirect roadway hazard impacts because the proposed project does not require or induce the construction of roadway design features.  PAR 1122 simply delays an exemption for affected equipment, so it is expected that the operation of the small batch-loaded cold cleaners and vapor degreasers would not change.

XVII. g): Affected facilities would still be expected to comply with, and not interfere with adopted policies, plans, or programs supporting alternative transportation.  Delay of the exemption for small batch-loaded cold cleaners and vapor degreasers from the requirements of Rule 1122 will not hinder compliance with any applicable alternative transportation plans or policies because the exempt activities are not associated transportation plans or policies.

Based on the above considerations, significant adverse impacts to transportation/circulation are not expected from PAR 1122.  Since there are no significant adverse impacts, no mitigation measures are required.

|  | Potentially Significant Impact | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- |
| XVIII. MANDATORY FINDINGS OF SIGNIFICANCE. |  |  |  |
| a) Does the project have the potential to degrade the quality of the environment, substantially reduce the habitat of a fish or wildlife species, cause a fish or wildlife population to drop below self-sustaining levels, threaten to eliminate a plant or animal community, reduce the number or restrict the range of a rare or endangered plant or animal or eliminate important examples of the major periods of California history or prehistory? |  |  |  |
| - Does the project have impacts that are individually limited, but cumulatively considerable? ("Cumulatively considerable" means that the incremental effects of a project are considerable when viewed in connection with the effects of past projects, the effects of other current projects, and the effects of probable future projects) |  |  |  |
| c) Does the project have environmental effects that will cause substantial adverse effects on human beings, either directly or indirectly? |  |  |  |

# Discussion

XVIII. a): As discussed in items I through XVII above, the proposed amended rule has no potential to cause significant adverse environmental effects because it would allow continued operation of small batch-loaded cold cleaners and vapor degreasers at existing facilities.  Therefore, the proposed project is not expected to degrade the quality of the environment, substantially reduce the habitat of a fish or wildlife species, cause a fish or wildlife population to drop below self sustaining levels, threaten to eliminate a plant or animal community, reduce the number or restrict the range of a rare or endangered plant or animal.  Similarly, PAR 1122 would not eliminate important examples of the major periods of California history or prehistory or otherwise degrade cultural resources.

XVIII.b)  Based on the foregoing analyses, since PAR 1122 will not result in project-specific significant environmental impacts, PAR 1122 is not expected to cause cumulative impacts in conjunction with other projects that may occur concurrently with or subsequent to the proposed project.  Furthermore, PAR 1122 impacts will not be "cumulatively considerable" because the incremental impacts are not considerable when viewed in connection with the effects of past, current, or probable future projects.

XVIII.c)  Based on the foregoing analyses, PAR 1122 is not expected to cause significant adverse effects on human beings, either directly, or indirectly.

A P P E N D I X   A

P R O P O S E D   A M E N D E D   R U L E   1 1 2 2

In order to save space and avoid repetition, please refer to the latest version of the proposed amended Rule 1122 located elsewhere in the rule package.  The version of the proposed amended rule circulated with the Draft EA was released on July 29, 2004 for a 30-day public review and comment period ending August 27, 2004.

Original hard copies of the Draft EA, which include the proposed amended rule at that time, can be obtained through the SCAQMD Public Information Center at the Diamond Bar headquarters or by calling (909) 396-2039.

A P P E N D I X   B

C O M M E N T S   O N   T H E   D R A F T   E A   A N D

R E S P O N S E S   T O   T H E   C O M M E N T S

C O M M E N T   L E T T E R   1

Boeing Satellite Systems

August 27, 2004

1-1

1-2

1-3

1-4

1-5

COMMENT LETTER # 1

FROM Boeing Satellite Systems

Michael A. Beasley

### August 27, 2004

Response to Comment 1-1:

As noted by the commentator, the existing exemption which is being maintained currently only applies to vapor degreasers.  The proposed amendments, however, will allow an exemption from rule provisions for small batch-loaded cold cleaners and small (less than one square foot) vapor degreasers as long as the equipment complies with the same provisions as the existing exemption for vapor degreasers.  The current exemption for small batch-loaded cold cleaners that limits the solvent usage at five gallons per calendar month will expire January 1, 2005.  The estimated five pounds per day, as disclosed in the Draft EA, from the continuation and expansion of this exemption is not expected to change and, therefore, the anticipated impacts from the proposed project will not change.  The project description in the Final EA will be updated to reflect this correction.

Response to Comment 1-2:

The affected equipment includes both small batch-loaded cold cleaners and vapor degreasers.  Vapor degreasers will continue to be limited to 22 pounds per month per facility.  The small batch-loaded cold cleaners, however, are currently limited to five gallons per calendar month which could translate to more than 22 pounds per month depending upon the VOC content of the solvent used in the equipment.  Thus, some facilities operating small batch-loaded cold cleaners will have to reduce their solvent usage in order to decrease their VOC emissions in order to qualify for the exemption.  The analysis does not take credit for the potential decrease in VOC emissions from the inclusion of the small batch-loaded cold cleaners in the exemption currently applied to vapor degreasers.  Thus, the statement in the Draft EA that “no increase in VOC emissions from the affected equipment is anticipated” remains true.  The statement “affected equipment will continue to be limited to 22 pounds per month” will be updated in the Final EA to delete “continue to” so it will read “affected equipment will be limited to 22 pounds per month.”  This change does not alter the original conclusions made in the Draft EA that the impact will not be significant.

Response to Comment 1-3:

It is correct that the increased amount of solvent is used due to the increased air flow from the operation of a control device.  The impact analysis in the Draft EA reached the same conclusion which can be found under the “Hazards and Hazardous Materials” discussion section that reads “emissions from affected facilities will not increase except in situations where operators install control equipment.”  No change to the Final EA to reflect this comment is necessary.

Response to Comment 1-4:

The use of carbon absorbers would increase the amount of waste generated from spent carbon.  However, the handling practices or procedure to remove, transport and regenerate the carbon is not anticipated to change from what is currently being done with other carbon absorption applications at a facility.  While a review and modification of a facility’s current hazardous waste handling practice or procedure might be necessary to manage the new amount of waste generated, it is not anticipated to substantially change if the facility already complies with established federal, state, and local statutes and regulations related to solid and hazardous waste handling and disposal.  Even if changes to the handling procedures are necessary for some facilities, the impact would still not be significant because of the minimal number of affected facilities expected to install control equipment.  Therefore, there will be no change to the original conclusions made in the Draft EA.

Response to Comment 1-5:

The “Solid/Hazardous Waste” section of the Draft EA was addressing the impacts from the general use of carbon filters and the typical process in regenerating the saturated carbon for active use.  SCAQMD recognizes that different facilities will use different solvents in their operations which will require different control equipment procedures.  Since isopropanol (also known as isopropyl alcohol or IPA) is classified as hazardous waste according the Resource Conservation and Recovery Act (RCRA) due to its low flashpoint of 53oF, thus, flammable, it would not be oxidized at a higher temperature as stated in the Draft EA.  After being forced out of the carbon bed by steam, IPA would most likely be sent through a desiccant in order to result in a clean alcohol byproduct, which would not be considered a solid or hazardous waste.  Any remaining IPA that is not further treated would still be considered hazardous waste.  In addition, any small amount of ash generated by the regeneration process will need to be disposed of.  The conclusion that the proposed project will result in a non-significant impact from solid/hazardous waste will not change since the amount of remaining hazardous waste to be disposed would be very small and, therefore, not exceed the capacity of designated landfills.

Open-top surface area less than one square foot or a capacity of less than two gallons.