APPENDIX A

DEMOLITION CALCULATIONS

| Construction Equipment Horsepower, Load Factors and Emission Factors - Demolition |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Emission Factors (d) |  |  |  |  |  |  |  |  |  |  |  |
| Equipment Type (a) | Fuel (a) | Horse-power (b) | Load Factor Percent (c) | CO lb/bhp-hr | VOC lb/bhp-hr | NOX lb/bhp-hr | SOX lb/bhp-hr | PM10 lb/bhp-hr | Hours of Operation hr/day (a) | Days of Operation day/wk (a) | Total Days of Operation (a) |
| 35-Ton Crane | Diesel | 212 | 43 | 0.009 | 0.003 | 0.023 | 0.002 | 0.0015 | 5 | 4 | 8 |
| 10-Ton Forklift | Diesel | 93 | 47.5 | 0.022 | 0.003 | 0.018 | 0.002 | 0.0015 | 5 | 4 | 8 |
| Generator | Diesel | 22 | 74 | 0.011 | 0.002 | 0.018 | 0.002 | 0.001 | 8 | 4 | 8 |
| Compressor | Diesel | 37 | 48 | 0.011 | 0.002 | 0.018 | 0.002 | 0.001 | 3 | 4 | 8 |
| Acetylene Torches (e) | Acetylene | NA | NA | NA | NA | NA | NA | NA | 8 | 4 | 8 |
| Exhaust Emissions (lb/day) = EF x BHP x (LF/100) x TH x N (f) |  |  |  |  |  |  |  |  |  |  |  |
|  | EF = | Emission factor for a specific air contaminant (lb/bhp-hr) |  |  |  |  |  |  |  |  |  |
|  | BHP = | Equipment brake horse power (bhp) |  |  |  |  |  |  |  |  |  |
|  | LF = | Equipment load factor |  |  |  |  |  |  |  |  |  |
|  | TH = | Equipment operating hours/day |  |  |  |  |  |  |  |  |  |
|  | N = | Number of pieces of equipment (one of each piece) |  |  |  |  |  |  |  |  |  |

| Construction Equipment Daily Emissions (lb/day) - Demolition |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Equipment Type | CO (lb/day) | VOC (lb/day) | NOX (lb/day) | SOX (lb/day) | PM10 (lb/day) |
| 35-Ton Crane | 4.10 | 1.37 | 10.48 | 0.91 | 0.68 |
| 10-Ton Forklift | 4.86 | 0.66 | 3.98 | 0.44 | 0.33 |
| Generator | 1.43 | 0.26 | 2.34 | 0.26 | 0.13 |
| Compressor | 0.59 | 0.11 | 0.96 | 0.11 | 0.05 |
| Acetylene Torches | NA | NA | NA | NA | NA |
| Totals: | 10.98 | 2.40 | 17.76 | 1.72 | 1.20 |

(a) E-mail from Tim Conkin (LADWP) to Dr. Krishna Nand (Parsons) dated April 21, 2003.

(b) Table A9-8-C, SCAQMD CEQA Handbook, 1993.

(c) Table A9-8-D, SCAQMD CEQA Handbook, 1993.

(d) Table A9-8-B, SCAQMD CEQA Handbook, 1993.

(e) No published emission factors for Acetylene Torches were found.  Emissions assumed to be negligible.

(f) Table A9-8, SCAQMD CEQA Handbook, 1993.

| Demolition Emissions from Construction Equipment and Motor Vehicles (Unpaved Surfaces) |  |  |  |
| --- | --- | --- | --- |
| Parameters Used to Calculate Onsite and Offsite Fugitive Dust PM10 Emissions |  |  |  |
| Parameter | Value | Basis |  |
| Soil silt content (s) | 7.5 percent | Table A9-9-E-1, SCAQMD 1993 CEQA Air Quality Handbook, Overburden |  |
| Soil moisture content (M) | 5.9 percent | "Open Fugitive Dust PM10 Control Strategies Study", Midwest Research Institute, October 12, 1990. |  |
| Construction equipment speed on unpaved surfaces (S) | 5 mph | Assumption |  |
| Material haul and delivery truck speeds on unpaved surfaces (S) | 5 mph | Assumption |  |
| Onsite pickup truck speed on unpaved surfaces (S) | 15 mph | Assumption |  |
| Watering Truck | 15 mph | Assumption |  |
| Construction equipment weight (W) | 30 tons | Estimated weight for heavy equipment |  |
| Material haul and delivery truck weight (W) | 40 tons | EIR |  |
| Onsite pickup truck weight (W) | 5 tons | Assumption |  |
| Watering Truck | 20 tons | Assumption |  |
| Emissions (lb/day) = | k x (S/15) x (s/12)a x (W/3)b / (M/0.2)c x VMT x N |  |  |
| S = | Equipment/motor vehicle speed (miles/hour) |  |  |
| s = | Soil silt content (percent) |  |  |
| W = | Equipment/motor vehicle weight (tons) |  |  |
| M = | Soil moisture (percent) |  |  |
| VMT = | Vehicle distance traveled (miles/vehicle-day) |  |  |
| N = | Number of vehicles |  |  |
| k = | 2.6 | Constant for Equation for PM10 factor |  |
| a = | 0.8 | Constant for Equation for PM10 factor |  |
| b = | 0.4 | Constant for Equation for PM10 factor |  |
| c = | 0.3 | Constant for Equation for PM10 factor |  |
| Equation 1, Section 13.2.2, U.S. EPA Compilation of Air Pollutant Emission Factors (AP-42), September 1998. |  |  |  |

Demolition Emissions from Construction Equipment and Motor Vehicles (Unpaved Surfaces)  -  (continued)

| Emissions (lb/day) = | k x (S/15) x (s/12)a x (W/3)b / (M/0.2)c x VMT x N |  |  |
| --- | --- | --- | --- |
| S = | 8.33 |  |  |
| s = | 0.075 |  |  |
| W = | 26.11 |  |  |
| M = | 0.059 |  |  |
| VMT = | 4.56 |  |  |
| N = | 9.00 |  |  |
| k = | 2.6 |  |  |
| a = | 0.8 |  |  |
| b = | 0.4 |  |  |
| c = | 0.3 |  |  |
| Emissions (lb/day) = | 3.50 |  |  |

| Demolition Emissions from Paved Road Dust Entrainment |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Emissions (g/day) = | k x (sL/2)0.65 x (W/3)1.5 x VMT |  |  |  |  |  |  |  |
| k = | particle size multiplier (used to compute PM10) |  |  |  |  |  |  |  |
| sL = | roadway silt loading in grams/m2 |  |  |  |  |  |  |  |
| W = | average weight of vehicles traveling the road (tons) |  |  |  |  |  |  |  |
| VMT = | Vehicle distance traveled (miles/vehicle-day) |  |  |  |  |  |  |  |
| California Air Resources Board Emission Inventory Methodology 7.9, Entrained Paved Road Dust (1997) |  |  |  |  |  |  |  |  |
| k (g/VMT) = | 7.3 |  |  |  |  |  |  |  |
| sL = | 0.037 |  |  |  |  |  |  |  |
| W = | 18.42 |  |  |  |  |  |  |  |
| VMT = | 40.00 |  |  |  |  |  |  |  |
| Emissions (g/day) = | 332 |  |  |  |  |  |  |  |
| Emissions (lb/day) = | 0.73 |  |  |  |  |  |  |  |

| Motor Vehicle Emissions During Demolition |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO and NOX Emissions (lbs/day) |  | [(EFRun x VMT) + (EFStart x Start)] x N / 454 |  |  |  |  |  |  |  |  |  |  |
| VOC Emissions (lb/day) = |  | [(EFRun x VMT) = (EFStart x Start) + (EFSoak x Trip) + (EFRest x Rest) + EFRunevap x VMT) + (EFDiurnal x Diurnal)] x N / 454 |  |  |  |  |  |  |  |  |  |  |
| PM10 Emissions (lb/day) = |  | [(EFRun + EFTire + EFBrake) x VMT + (EFStart x Start)] x N / 454 |  |  |  |  |  |  |  |  |  |  |
| EFRun = |  | Running exhaust emission factor (g/mi) |  |  |  |  |  |  |  |  |  |  |
| EFStart = |  | Start-up emission factor (g/start) |  |  |  |  |  |  |  |  |  |  |
| VMT = |  | Distance traveled (mi/vehicle-day) |  |  |  |  |  |  |  |  |  |  |
| Start = |  | Number of starts/vehicle-day |  |  |  |  |  |  |  |  |  |  |
| EFSoak = |  | Hot-soak emission factor (g/trip) |  |  |  |  |  |  |  |  |  |  |
| Trip = |  | One-way trips/vehicle-day |  |  |  |  |  |  |  |  |  |  |
| EFRest = |  | Resting loss evaporative emission factor (g/hr) |  |  |  |  |  |  |  |  |  |  |
| Rest = |  | Resting time with constant or decreasing ambient temperature (hour/vehicle-day) |  |  |  |  |  |  |  |  |  |  |
| EFRunevap = |  | Running evaporative emission factor (g/mi) |  |  |  |  |  |  |  |  |  |  |
| EFDiurnal = |  | Diurnal evaporative emission factor |  |  |  |  |  |  |  |  |  |  |
| Diurnal = |  | Time with increasing ambient temperature (hours/vehicle-day) |  |  |  |  |  |  |  |  |  |  |
| EFTire = |  | Time wear emission factor (g/mi) |  |  |  |  |  |  |  |  |  |  |
| EFBrake = |  | Break wear emission factor (g/mi) |  |  |  |  |  |  |  |  |  |  |
| N = |  | Number of Vehicles |  |  |  |  |  |  |  |  |  |  |
|  |  | Vehicle Type | CO (lb/day) |  | VOC (lb/day) |  | NOX (lb/day) | PM10 (lb/day) |  |  |  |  |
|  |  | Pickup Truck, Onsite | 0.98 |  | 0.11 |  | 0.10 | 0.00 |  |  |  |  |
|  |  | Delivery vehicle, onsite | 0.09 |  | 0.02 |  | 0.16 | 0.01 |  |  |  |  |
|  |  | Watering Truck, Onsite | 0.02 |  | 0.00 |  | 0.04 | 0.00 |  |  |  |  |
|  |  | Material/Haul Truck, onsite | 0.06 |  | 0.01 |  | 0.11 | 0.01 |  |  |  |  |
|  |  | Construction Commuter | 5.54 |  | 0.34 |  | 0.45 | 0.02 |  |  |  |  |
|  |  | Deliver vehicle, offsite | 1.08 |  | 0.30 |  | 3.81 | 0.15 |  |  |  |  |
|  |  | Material/Haul Truck, offsite | 0.72 |  | 0.20 |  | 2.54 | 0.10 |  |  |  |  |
|  |  | Totals: | 8.48 |  | 0.99 |  | 7.20 | 0.28 |  |  |  |  |
| Motor Vehicle Classes, Speeds and Daily VMT During Demolition |  |  |  |  |  |  |  |  |  |  |  |  |
| Vehicle Type | Vehicle Class |  | Speed (mph) |  | VMT (mi/vehicle-day) |  | # of Starts | # of Trips | Rest Hours | Diurnal Hours | # of Vehicles |  |
| Pickup Truck, Onsite | Medium duty truck, catalyst |  | 15 |  | 10 |  | 2 | 2 | 12 | 12 | 3 |  |
| Delivery vehicle, onsite | Heavy heavy-duty truck, diesel |  | 5 |  | 1 |  | NA | NA | NA | NA | 3 |  |
| Watering Truck, Onsite | Heavy heavy-duty truck, diesel |  | 15 |  | 1 |  | NA | NA | NA | NA | 1 |  |
| Material/Haul Truck, onsite | Heavy heavy-duty truck, diesel |  | 5 |  | 1 |  | NA | NA | NA | NA | 2 |  |
| Construction Commuter | Light-Duty truck, catalyst |  | 35 |  | 40 |  | 2 | 2 | 12 | 12 | 7 |  |
| Deliver vehicle, offsite | Heavy heavy-duty truck, diesel |  | 25 |  | 40 |  | NA | NA | NA | NA | 3 |  |
| Material/Haul Truck, offsite | Heavy heavy-duty truck, diesel |  | 25 |  | 40 |  | NA | NA | NA | NA | 2 |  |

| Motor Vehicle CO and NOX Emission Factors During Demolition |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicle Type | CO |  | NOX |  |  |  |  |  |
|  | Running Exhaust (g/mi) | Start-Up (g/start)(a) | Running Exhaust (g/mi) | Start-Up (g/start)a |  |  |  |  |
| Pickup Truck, Onsite | 10.029 | 24.146 | 1.326 | 1.224 |  |  |  |  |
| Delivery vehicle, onsite | 13.709 | NA | 24.559 | NA |  |  |  |  |
| Watering Truck, Onsite | 6.821 | NA | 17.515 | NA |  |  |  |  |
| Material/Haul Truck, onsite | 13.709 | NA | 24.559 | NA |  |  |  |  |
| Construction Commuter | 8.066 | 18.351 | 0.679 | 0.979 |  |  |  |  |
| Deliver vehicle, offsite | 4.072 | NA | 14.398 | NA |  |  |  |  |
| Material/Haul Truck, offsite | 4.072 | NA | 14.398 | NA |  |  |  |  |
| (a) - Assumed to be after 720 minutes with engine off |  |  |  |  |  |  |  |  |
| Source: ARB EMFAC2002 motor vehicle emission factor model, version 2.2, for calendar year 2003, summer |  |  |  |  |  |  |  |  |

| Motor Vehicle VOC Emission Factors During Demolition |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicle Type | Running Exhaust (g/mi) | Start-Up (g/start)(a) | Hot-Soak (g/trip) | Resting Loss (g/hr) | Running Evaporative (g/mi) | Diurnal Evaporative (g/hr) |  |  |
| Pickup Truck, Onsite | 0.747 | 2.756 | 0.27 | 0.068 | 0.055 | 0.191 |  |  |
| Delivery vehicle, onsite | 2.636 | NA | NA | NA | NA | NA |  |  |
| Watering Truck, Onsite | 1.661 | NA | NA | NA | NA | NA |  |  |
| Material/Haul Truck, onsite | 2.636 | NA | NA | NA | NA | NA |  |  |
| Construction Commuter | 0.275 | 2.082 | 0.3 | 0.082 | 0.062 | 0.225 |  |  |
| Deliver vehicle, offsite | 1.143 | NA | NA | NA | NA | NA |  |  |
| Material/Haul Truck, offsite | 1.143 | NA | NA | NA | NA | NA |  |  |
| (a) - Assumed to be after 720 minutes with engine off |  |  |  |  |  |  |  |  |
| Source: ARB EMFAC2002 motor vehicle emission factor model, version 2.2, for calendar year 2003, summer |  |  |  |  |  |  |  |  |

| Motor Vehicle PM10 Emission Factors During Demolition |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicle Type | Running Exhaust (g/mi) | Tire Wear (g/mi) | Brake Wear (g/mi) | Start-Up (g/start)(a) |  |  |  |  |
| Pickup Truck, Onsite | 0.045 | 0.008 | 0.013 | 0.026 |  |  |  |  |
| Delivery vehicle, onsite | 1.115 | 0.036 | 0.013 | NA |  |  |  |  |
| Watering Truck, Onsite | 0.728 | 0.036 | 0.013 | NA |  |  |  |  |
| Material/Haul Truck, onsite | 1.115 | 0.036 | 0.013 | NA |  |  |  |  |
| Construction Commuter | 0.012 | 0.008 | 0.013 | 0.016 |  |  |  |  |
| Deliver vehicle, offsite | 0.501 | 0.036 | 0.013 | NA |  |  |  |  |
| Material/Haul Truck, offsite | 0.501 | 0.036 | 0.013 | NA |  |  |  |  |
| (a) - Assumed to be after 720 minutes with engine off |  |  |  |  |  |  |  |  |
| Source: ARB EMFAC2002 motor vehicle emission factor model, version 2.2, for calendar year 2003, summer |  |  |  |  |  |  |  |  |

APPENDIX B

CONSTRUCTION AND POST-CONSTRUCTION CALCULATIONS

| Construction Equipment Horsepower, Load Factors and Emission Factors – Tank Construction |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Emission Factors (d) |  |  |  |  |  |  |  |  |  |  |  |
| Equipment Type (a) | Fuel (a) | Horse-power (b) | Load Factor Percent (c) | CO lb/bhp-hr | VOC lb/bhp-hr | NOX lb/bhp-hr | SOX lb/bhp-hr | PM10 lb/bhp-hr | Hours of Operation hr/day (a) | Days of Operation day/wk (a) | Total Days of Operation (a) |
| 35-Ton Crane | Diesel | 212 | 43 | 0.009 | 0.003 | 0.023 | 0.002 | 0.0015 | 5 | 4 | 80 |
| 10-Ton Forklift | Diesel | 93 | 47.5 | 0.022 | 0.003 | 0.018 | 0.002 | 0.0015 | 5 | 4 | 80 |
| Generator | Diesel | 22 | 74 | 0.011 | 0.002 | 0.018 | 0.002 | 0.001 | 8 | 4 | 80 |
| Compressor | Diesel | 37 | 48 | 0.011 | 0.002 | 0.018 | 0.002 | 0.001 | 3 | 4 | 80 |
| Acetylene Torches (e) | Acetylene | NA | NA | NA | NA | NA | NA | NA | 8 | 4 | 80 |
| Exhaust Emissions (lb/day) = EF x BHP x (LF/100) x TH x N (f) |  |  |  |  |  |  |  |  |  |  |  |
|  | EF = | Emission factor for a specific air contaminant (lb/bhp-hr) |  |  |  |  |  |  |  |  |  |
|  | BHP = | Equipment brake horse power (bhp) |  |  |  |  |  |  |  |  |  |
|  | LF = | Equipment load factor |  |  |  |  |  |  |  |  |  |
|  | TH = | Equipment operating hours/day |  |  |  |  |  |  |  |  |  |
|  | N = | Number of pieces of equipment (one of each piece) |  |  |  |  |  |  |  |  |  |

| Construction Equipment Daily Emissions (lb/day) – Tank Construction |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Equipment Type | CO (lb/day) | VOC (lb/day) | NOX (lb/day) | SOX (lb/day) | PM10 (lb/day) |
| 35-Ton Crane | 4.10 | 1.37 | 10.48 | 0.91 | 0.68 |
| 10-Ton Forklift | 4.86 | 0.66 | 3.98 | 0.44 | 0.33 |
| Generator | 1.43 | 0.26 | 2.34 | 0.26 | 0.13 |
| Compressor | 0.59 | 0.11 | 0.96 | 0.11 | 0.05 |
| Acetylene Torches | NA | NA | NA | NA | NA |
| Totals: | 10.98 | 2.40 | 17.76 | 1.72 | 1.20 |

(a) E-mail from Tim Conkin (LADWP) to Dr. Krishna Nand (Parsons) dated April 21, 2003.

(b) Table A9-8-C, SCAQMD CEQA Handbook, 1993.

(c) Table A9-8-D, SCAQMD CEQA Handbook, 1993.

(d) Table A9-8-B, SCAQMD CEQA Handbook, 1993.

(e) No published emission factors for Acetylene Torches were found.  Emissions assumed to be negligible.

(f) Table A9-8, SCAQMD CEQA Handbook, 1993.

| Tank Construction Emissions from Construction Equipment and Motor Vehicles (Unpaved Surfaces) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameters Used to Calculate Onsite and Offsite Fugitive Dust PM10 Emissions |  |  |  |  |
| Parameter | Value | Basis |  |  |
| Soil silt content (s) | 7.5 percent | Table A9-9-E-1, SCAQMD 1993 CEQA Air Quality Handbook, Overburden |  |  |
| Soil moisture content (M) | 5.9 percent | “Open Fugitive Dust PM10 Control Strategies Study”, Midwest Research Institute, October 12, 1990. |  |  |
| Construction equipment speed on unpaved surfaces (S) | 5 mph | Assumption |  |  |
| Material haul and delivery truck speeds on unpaved surfaces (S) | 5 mph | Assumption |  |  |
| Onsite pickup truck speed on unpaved surfaces (S) | 15 mph | Assumption |  |  |
| Watering Truck | 15 mph | Assumption |  |  |
| Construction equipment weight (W) | 30 tons | Estimated weight for heavy equipment |  |  |
| Material haul and delivery truck weight (W) | 40 tons | EIR |  |  |
| Onsite pickup truck weight (W) | 5 tons | Assumption |  |  |
| Watering Truck | 20 tons | Assumption |  |  |
| Emissions (lb/day) = | k x (S/15) x (s/12)a x (W/3)b / (M/0.2)c x VMT x N |  |  |  |
| S = | Equipment/motor vehicle speed (miles/hour) |  |  |  |
| s = | Soil silt content (percent) |  |  |  |
| W = | Equipment/motor vehicle weight (tons) |  |  |  |
| M = | Soil moisture (percent) |  |  |  |
| VMT = | Vehicle distance traveled (miles/vehicle-day) |  |  |  |
| N = | Number of vehicles |  |  |  |
| k = | 2.6 | Constant for Equation for PM10 factor |  |  |
| a = | 0.8 | Constant for Equation for PM10 factor |  |  |
| b = | 0.4 | Constant for Equation for PM10 factor |  |  |
| c = | 0.3 | Constant for Equation for PM10 factor |  |  |
| Equation 1, Section 13.2.2, U.S. EPA Compilation of Air Pollutant Emission Factors (AP-42), September 1998. |  |  |  |  |

| Tank Construction Emissions from Construction Equipment and Motor Vehicles (Unpaved Surfaces) - (continued) |  |  |  |
| --- | --- | --- | --- |
| Emissions (lb/day) = | k x (S/15) x (s/12)a x (W/3)b / (M/0.2)c x VMT x N |  |  |
| S = | 16.11 |  |  |
| s = | 0.075 |  |  |
| W = | 50.56 |  |  |
| M = | 0.059 |  |  |
| VMT = | 5.67 |  |  |
| N = | 15.00 |  |  |
| k = | 2.6 |  |  |
| a = | 0.8 |  |  |
| b = | 0.4 |  |  |
| c = | 0.3 |  |  |
| Emissions (lb/day) = | 18.27 |  |  |

| Tank Construction Emissions from Paved Road Dust Entrainment |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Emissions (g/day) = | k x (sL/2)0.65 x (W/3)1.5 x VMT |  |  |  |  |  |  |  |
| k = | particle size multiplier (used to compute PM10) |  |  |  |  |  |  |  |
| sL = | roadway silt loading in grams/m2 |  |  |  |  |  |  |  |
| W = | average weight of vehicles traveling the road (tons) |  |  |  |  |  |  |  |
| VMT = | Vehicle distance traveled (miles/vehicle-day) |  |  |  |  |  |  |  |
| California Air Resources Board Emission Inventory Methodology 7.9, Entrained Paved Road Dust (1997) |  |  |  |  |  |  |  |  |
| k (g/VMT) = | 7.3 |  |  |  |  |  |  |  |
| sL = | 0.037 |  |  |  |  |  |  |  |
| W = | 23.81 |  |  |  |  |  |  |  |
| VMT = | 40.00 |  |  |  |  |  |  |  |
| Emissions (g/day) = | 488 |  |  |  |  |  |  |  |
| Emissions (lb/day) = | 1.08 |  |  |  |  |  |  |  |

| Tank Construction Motor Vehicle Emissions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO and NOX Emissions (lb/day) | [(EFRun x VMT) + (EFStart x Start)] x N / 454 |  |  |  |  |  |  |  |
| VOC Emissions (lb/day) | [(EFRun x VMT) = (EFStart x Start) + (EFSoak x Trip) + (EFRest x Rest) + EFRunevap x VMT) + (EFDiurnal x Diurnal)] x N / 454 |  |  |  |  |  |  |  |
| PM10 Emissions (lb/day) | [(EFRun + EFTire + EFBrake) x VMT + (EFStart x Start)] x N / 454 |  |  |  |  |  |  |  |
| EFRun = | Running exhaust emission factor (g/mi) |  |  |  |  |  |  |  |
| EFStart = | Start-up emission factor (g/start) |  |  |  |  |  |  |  |
| VMT = | Distance traveled (mi/vehicle-day) |  |  |  |  |  |  |  |
| Start = | Number of starts/vehicle-day |  |  |  |  |  |  |  |
| EFSoak = | Hot-soak emission factor (g/trip) |  |  |  |  |  |  |  |
| Trip = | One-way trips/vehicle-day |  |  |  |  |  |  |  |
| EFRest = | Resting loss evaporative emission factor (g/hr) |  |  |  |  |  |  |  |
| Rest = | Resting time with constant or decreasing ambient temperature (hour/vehicle-day) |  |  |  |  |  |  |  |
| EFRunevap = | Running evaporative emission factor (g/mi) |  |  |  |  |  |  |  |
| EFDiurnal = | Diurnal evaporative emission factor |  |  |  |  |  |  |  |
| Diurnal = | Time with increasing ambient temperature (hours/vehicle-day) |  |  |  |  |  |  |  |
| EFTire = | Time wear emission factor (g/mi) |  |  |  |  |  |  |  |
| EFBrake = | Break wear emission factor (g/mi) |  |  |  |  |  |  |  |
| N = | Number of Vehicles |  |  |  |  |  |  |  |

|  | Vehicle Type | CO (lb/day) | VOC (lb/day) | NOX (lb/day) | PM10 (lb/day) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pickup Truck, Onsite | 0.98 | 0.11 | 0.10 | 0.00 |  |  |  |
|  | Delivery vehicle, onsite | 0.09 | 0.02 | 0.16 | 0.01 |  |  |  |
|  | Watering Truck, Onsite | 0.02 | 0.00 | 0.04 | 0.00 |  |  |  |
|  | Material/Haul Truck, onsite | 0.18 | 0.03 | 0.32 | 0.02 |  |  |  |
|  | Construction Commuter | 5.54 | 0.34 | 0.45 | 0.02 |  |  |  |
|  | Deliver vehicle, offsite | 1.08 | 0.30 | 3.81 | 0.15 |  |  |  |
|  | Material/Haul Truck, offsite | 2.15 | 0.60 | 7.61 | 0.29 |  |  |  |
|  | Totals: | 10.04 | 1.41 | 12.50 | 0.49 |  |  |  |

| Motor Vehicle Classes, Speeds and Daily VMT During Tank Construction |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicle Type | Vehicle Class | Speed (mph) | VMT (mi/vehicle-day) | # of Starts | # of Trips | Rest Hours | Diurnal Hours | # of Vehicles |
| Pickup Truck, Onsite | Medium duty truck, catalyst | 15 | 10 | 2 | 2 | 12 | 12 | 3 |
| Delivery vehicle, onsite | Heavy heavy-duty truck, diesel | 5 | 1 | NA | NA | NA | NA | 3 |
| Watering Truck, Onsite | Heavy heavy-duty truck, diesel | 15 | 1 | NA | NA | NA | NA | 1 |
| Material/Haul Truck, onsite | Heavy heavy-duty truck, diesel | 5 | 1 | NA | NA | NA | NA | 6 |
| Construction Commuter | Light-Duty truck, catalyst | 35 | 40 | 2 | 2 | 12 | 12 | 7 |
| Deliver vehicle, offsite | Heavy heavy-duty truck, diesel | 25 | 40 | NA | NA | NA | NA | 3 |
| Material/Haul Truck, offsite | Heavy heavy-duty truck, diesel | 25 | 40 | NA | NA | NA | NA | 6 |

| Motor Vehicle CO and NOX Emission Factors During Tank Construction (summer months) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicle Type |  | CO |  |  |  | NOX |  |  |  |  |
|  |  | Running Exhaust (g/mi) |  | Start-Up (g/start)(a) |  | Running Exhaust (g/mi) |  |  | Start-Up (g/start)a |  |
| Pickup Truck, Onsite |  | 10.029 |  | 24.146 |  | 1.326 |  |  | 1.224 |  |
| Delivery vehicle, onsite |  | 13.709 |  | NA |  | 24.559 |  |  | NA |  |
| Watering Truck, Onsite |  | 6.821 |  | NA |  | 17.515 |  |  | NA |  |
| Material/Haul Truck, onsite |  | 13.709 |  | NA |  | 24.559 |  |  | NA |  |
| Construction Commuter |  | 8.066 |  | 18.351 |  | 0.679 |  |  | 0.979 |  |
| Deliver vehicle, offsite |  | 4.072 |  | NA |  | 14.398 |  |  | NA |  |
| Material/Haul Truck, offsite |  | 4.072 |  | NA |  | 14.398 |  |  | NA |  |
| (a) - Assumed to be after 720 minutes with engine off |  |  |  |  |  |  |  |  |  |  |
| Source: ARB EMFAC2002 motor vehicle emission factor model, version 2.2, for calendar year 2003, summer |  |  |  |  |  |  |  |  |  |  |

| Motor Vehicle CO and NOX Emission Factors During Tank Construction (October) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Vehicle Type | CO |  | NOX |  |  |
|  | Running Exhaust (g/mi) | Start-Up (g/start)(a) | Running Exhaust (g/mi) | Start-Up (g/start)(a) |  |
| Pickup Truck, Onsite | 9.269 | 24.662 | 1.458 | 1.305 |  |
| Delivery vehicle, onsite | 13.709 | NA | 25.664 | NA |  |
| Watering Truck, Onsite | 6.821 | NA | 18.303 | NA |  |
| Material/Haul Truck, onsite | 13.709 | NA | 25.664 | NA |  |
| Construction Commuter | 7.502 | 18.746 | 0.75 | 1.044 |  |
| Deliver vehicle, offsite | 4.072 | NA | 15.046 | NA |  |
| Material/Haul Truck, offsite | 4.072 | NA | 15.046 | NA |  |
| (a) - Assumed to be after 720 minutes with engine off |  |  |  |  |  |
| Source: ARB EMFAC2002 motor vehicle emission factor model, version 2.2, for calendar year 2003, October |  |  |  |  |  |

| Motor Vehicle CO and NOX Emission Factors During Tank Construction (November) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicle Type |  | CO |  |  |  | NOX |  |  |  |  |
|  |  | Running Exhaust (g/mi) |  | Start-Up (g/start)(a) |  | Running Exhaust (g/mi) |  |  | Start-Up (g/start)(a) |  |
| Pickup Truck, Onsite |  | 9.307 |  | 25.647 |  | 1.615 |  |  | 1.396 |  |
| Delivery vehicle, onsite |  | 13.709 |  | NA |  | 26.43 |  |  | NA |  |
| Watering Truck, Onsite |  | 6.821 |  | NA |  | 18.85 |  |  | NA |  |
| Material/Haul Truck, onsite |  | 13.709 |  | NA |  | 26.43 |  |  | NA |  |
| Construction Commuter |  | 7.532 |  | 19.481 |  | 0.032 |  |  | 1.117 |  |
| Deliver vehicle, offsite |  | 4.072 |  | NA |  | 15.495 |  |  | NA |  |
| Material/Haul Truck, offsite |  | 4.072 |  | NA |  | 15.495 |  |  | NA |  |
| (a) - Assumed to be after 720 minutes with engine off |  |  |  |  |  |  |  |  |  |  |
| Source: ARB EMFAC2002 motor vehicle emission factor model, version 2.2, for calendar year 2003, November |  |  |  |  |  |  |  |  |  |  |

| Motor Vehicle VOC Emission Factors During Tank Construction (summer months) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicle Type | Running Exhaust (g/mi) | Start-Up (g/start)(a) | Hot-Soak (g/trip) | Resting Loss (g/hr) | Running Evaporative (g/mi) | Diurnal Evaporative (g/hr) |  |  |
| Pickup Truck, Onsite | 0.747 | 2.756 | 0.27 | 0.068 | 0.055 | 0.191 |  |  |
| Delivery vehicle, onsite | 2.636 | NA | NA | NA | NA | NA |  |  |
| Watering Truck, Onsite | 1.661 | NA | NA | NA | NA | NA |  |  |
| Material/Haul Truck, onsite | 2.636 | NA | NA | NA | NA | NA |  |  |
| Construction Commuter | 0.275 | 2.082 | 0.3 | 0.082 | 0.062 | 0.225 |  |  |
| Deliver vehicle, offsite | 1.143 | NA | NA | NA | NA | NA |  |  |
| Material/Haul Truck, offsite | 1.143 | NA | NA | NA | NA | NA |  |  |
| (a) - Assumed to be after 720 minutes with engine off |  |  |  |  |  |  |  |  |
| Source: ARB EMFAC2002 motor vehicle emission factor model, version 2.2, for calendar year 2003, summer |  |  |  |  |  |  |  |  |

| Motor Vehicle VOC Emission Factors During Tank Construction (October) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicle Type | Running Exhaust (g/mi) | Start-Up (g/start)(a) | Hot-Soak (g/trip) | Resting Loss (g/hr) | Running Evaporative (g/mi) | Diurnal Evaporative (g/hr) |  |  |
| Pickup Truck, Onsite | 0.714 | 2.8 | 0.202 | 0.053 | 0.055 | 0.013 |  |  |
| Delivery vehicle, onsite | 2.636 | NA | NA | NA | NA | NA |  |  |
| Watering Truck, Onsite | 1.661 | NA | NA | NA | NA | NA |  |  |
| Material/Haul Truck, onsite | 2.636 | NA | NA | NA | NA | NA |  |  |
| Construction Commuter | 0.264 | 2.115 | 0.222 | 0.063 | 0.062 | 0.013 |  |  |
| Deliver vehicle, offsite | 1.143 | NA | NA | NA | NA | NA |  |  |
| Material/Haul Truck, offsite | 1.143 | NA | NA | NA | NA | NA |  |  |
| (a) - Assumed to be after 720 minutes with engine off |  |  |  |  |  |  |  |  |
| Source: ARB EMFAC2002 motor vehicle emission factor model, version 2.2, for calendar year 2003, October |  |  |  |  |  |  |  |  |

| Motor Vehicle VOC Emission Factors During Tank Construction (November) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicle Type | Running Exhaust (g/mi) | Start-Up (g/start)(a) | Hot-Soak (g/trip) | Resting Loss (g/hr) | Running Evaporative (g/mi) | Diurnal Evaporative (g/hr) |  |  |
| Pickup Truck, Onsite | 0.714 | 2.846 | 0.269 | 0.042 | 0.12 | 0.011 |  |  |
| Delivery vehicle, onsite | 2.636 | NA | NA | NA | NA | NA |  |  |
| Watering Truck, Onsite | 1.661 | NA | NA | NA | NA | NA |  |  |
| Material/Haul Truck, onsite | 2.636 | NA | NA | NA | NA | NA |  |  |
| Construction Commuter | 0.264 | 2.152 | 0.295 | 0.05 | 0.137 | 0.011 |  |  |
| Deliver vehicle, offsite | 1.143 | NA | NA | NA | NA | NA |  |  |
| Material/Haul Truck, offsite | 1.143 | NA | NA | NA | NA | NA |  |  |
| (a) - Assumed to be after 720 minutes with engine off |  |  |  |  |  |  |  |  |
| Source: ARB EMFAC2002 motor vehicle emission factor model, version 2.2, for calendar year 2003, November |  |  |  |  |  |  |  |  |
| Motor Vehicle PM10 Emission Factors During Tank Construction (summer months) |  |  |  |  |  |  |  |  |
| Vehicle Type | Running Exhaust (g/mi) | Tire Wear (g/mi) | Brake Wear (g/mi) | Start-Up (g/start)(a) |  |  |  |  |
| Pickup Truck, Onsite | 0.045 | 0.008 | 0.013 | 0.026 |  |  |  |  |
| Delivery vehicle, onsite | 1.115 | 0.036 | 0.013 | NA |  |  |  |  |
| Watering Truck, Onsite | 0.728 | 0.036 | 0.013 | NA |  |  |  |  |
| Material/Haul Truck, onsite | 1.115 | 0.036 | 0.013 | NA |  |  |  |  |
| Construction Commuter | 0.012 | 0.008 | 0.013 | 0.016 |  |  |  |  |
| Deliver vehicle, offsite | 0.501 | 0.036 | 0.013 | NA |  |  |  |  |
| Material/Haul Truck, offsite | 0.501 | 0.036 | 0.013 | NA |  |  |  |  |
| (a) - Assumed to be after 720 minutes with engine off |  |  |  |  |  |  |  |  |
| Source: ARB EMFAC2002 motor vehicle emission factor model, version 2.2, for calendar year 2003, summer |  |  |  |  |  |  |  |  |

| Motor Vehicle PM10 Emission Factors During Tank Construction (October) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicle Type | Running Exhaust (g/mi) | Tire Wear (g/mi) | Brake Wear (g/mi) | Start-Up (g/start)(a) |  |  |  |  |
| Pickup Truck, Onsite | 0.045 | 0.008 | 0.013 | 0.026 |  |  |  |  |
| Delivery vehicle, onsite | 1.115 | 0.036 | 0.013 | NA |  |  |  |  |
| Watering Truck, Onsite | 0.728 | 0.036 | 0.013 | NA |  |  |  |  |
| Material/Haul Truck, onsite | 1.115 | 0.036 | 0.013 | NA |  |  |  |  |
| Construction Commuter | 0.011 | 0.008 | 0.013 | 0.016 |  |  |  |  |
| Deliver vehicle, offsite | 0.501 | 0.036 | 0.013 | NA |  |  |  |  |
| Material/Haul Truck, offsite | 0.501 | 0.036 | 0.013 | NA |  |  |  |  |
| (a) - Assumed to be after 720 minutes with engine off |  |  |  |  |  |  |  |  |
| Source: ARB EMFAC2002 motor vehicle emission factor model, version 2.2, for calendar year 2003, October |  |  |  |  |  |  |  |  |
| Motor Vehicle PM10 Emission Factors During Tank Construction (November) |  |  |  |  |  |  |  |  |
| Vehicle Type | Running Exhaust (g/mi) | Tire Wear (g/mi) | Brake Wear (g/mi) | Start-Up (g/start)(a) |  |  |  |  |
| Pickup Truck, Onsite | 0.045 | 0.008 | 0.013 | 0.026 |  |  |  |  |
| Delivery vehicle, onsite | 1.115 | 0.036 | 0.013 | NA |  |  |  |  |
| Watering Truck, Onsite | 0.728 | 0.036 | 0.013 | NA |  |  |  |  |
| Material/Haul Truck, onsite | 1.115 | 0.036 | 0.013 | NA |  |  |  |  |
| Construction Commuter | 0.011 | 0.008 | 0.013 | 0.016 |  |  |  |  |
| Deliver vehicle, offsite | 0.501 | 0.036 | 0.013 | NA |  |  |  |  |
| Material/Haul Truck, offsite | 0.501 | 0.036 | 0.013 | NA |  |  |  |  |
| (a) - Assumed to be after 720 minutes with engine off |  |  |  |  |  |  |  |  |
| Source: ARB EMFAC2002 motor vehicle emission factor model, version 2.2, for calendar year 2003, November |  |  |  |  |  |  |  |  |

| Post-Construction Activities - Architectural Coatings (Tank) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Emissions (lbs) = | C*(T/12)*A/(TE/100)*E |  |  |  |  |  |
| C = | Constant/Conversion = 1 gallon = 0.133681 ft3 |  |  |  |  |  |
| T = | Thickness of the coating in mils (1 mil = 0.001 inches) |  |  |  |  |  |
| A = | Area to be coated (ft2) |  |  |  |  |  |
| TE = | Transfer Efficiency of HVLP Sprayer |  |  |  |  |  |
| E = | VOC content of the coating |  |  |  |  |  |
| Tank Surface Area |  |  |  |  |  |  |
| Bottom = | 99 | feet diameter |  |  |  |  |
| Area = | 7697.687 | square feet |  |  |  |  |
| Sides = | 48 | feet tall |  |  |  |  |
|  | 311.018 | length = circumference of 99 feet diameter circle |  |  |  |  |
|  | 14928.848 | square feet |  |  |  |  |
| Top = | 8009.000 | square feet |  |  |  |  |
| Interior Coating |  |  |  |  |  |  |
| Interior = bottom + 3' up shell |  |  |  |  |  |  |
| Interior = | 8630.740 | square feet |  |  |  |  |
| Exterior Coating |  |  |  |  |  |  |
| Exterior = Sides only |  |  |  |  |  |  |
| Exterior = | 22938.000 | square feet |  |  |  |  |
| Calculation: | C = 7.4805 | gal/ft3 |  |  |  |  |
|  | T | A | TE | E |  |  |
|  | thickness (inches) | ft2 | % | lbs VOC/gal | lbs |  |
| Interior - protectant | 0.012 | 8630.740 | 65 | 1.399 | 138.958 |  |
| Exterior – protectant r | 0.006 | 22938.000 | 65 | 0.179 | 23.626 |  |
| Exterior – sealer/finisher | 0.004 | 22938.000 | 65 | 1.173 | 103.217 |  |
| Total lbs. |  |  |  |  | 265.801 |  |
| Total number of days of Painting = |  | 24 | days |  | 11.075 | lbs/day |

APPENDIX C

OPERATIONAL FUGITIVE EMISSIONS CALCULATIONS

OPERATIONAL FUGITIVE EMISSIONS FROM PUMPS, VALVES, FLANGES

AND THREADED CONNECTORS

| Source | Number of Components | Emission Factor (lbs/year) | Fugitive (VOC) Emissions (lbs/year) | Fugitive (VOC) Emissions (lbs/day) |
| --- | --- | --- | --- | --- |
| Pumps | 5 | 80 | 400 | 1.10 |
| Valves | 90 | 3 | 270 | 0.74 |
| Flanges and Threaded Connectors | 314 | 1.5 | 471 | 1.30 |
| TOTAL DAILY EMISSIONS |  |  |  | 3.14 |

Note:  Fugitive emissions in pounds per day were derived by dividing the pounds per year by 365 days.

Source: SCAQMD Permit to Construct/Operate Application Evaluation and Calculations, dated August 22, 2003.

OPERATIONAL FUGITIVE EMISSIONS FROM 60,000 BARREL AST

| Source | Fugitive VOC Emissions (lbs/year) | Fugitive VOC Emissions (lbs/day) |
| --- | --- | --- |
| Rim Seal losses | 12.6230 | 0.035 |
| Withdrawal losses | 0.5690 | 0.002 |
| Deck Fitting losses | 14.9268 | 0.041 |
| Deck Seam losses | 0.0 | 0.0 |
| TOTAL DAILY EMISSIONS |  | 0.078 |

Note:  Fugitive emissions in pounds per day were derived by dividing the pounds per year by 365 days.

Source: SCAQMD Permit to Construct/Operate Application Evaluation and Calculations, dated August 22, 2003.

| Architectural Coating – Tank Construction |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Emissions (lbs) = | C*(T/12)*A/(TE/100)*E |  |  |  |  |  |
| C = | Constant/Conversion = 1 gallon = 0.133681 ft3 |  |  |  |  |  |
| T = | Thickness of the coating in mils (1 mil = 0.001 inches) |  |  |  |  |  |
| A = | Area to be coated (ft2) |  |  |  |  |  |
| TE = | Transfer Efficiency of HVLP Sprayer |  |  |  |  |  |
| E = | VOC content of the coating |  |  |  |  |  |
| Tank Surface Area |  |  |  |  |  |  |
| Bottom = | 99 | feet diameter |  |  |  |  |
| Area = | 7697.687 | square feet |  |  |  |  |
| Sides = | 48 | feet tall |  |  |  |  |
|  | 311.018 | length = circumference of 99 feet diameter circle |  |  |  |  |
|  | 14928.848 | square feet |  |  |  |  |
| Top = | 8009.000 | square feet |  |  |  |  |
| Interior Coating |  |  |  |  |  |  |
| Interior = | 8630.740 | square feet |  |  |  |  |
| Exterior Coating |  |  |  |  |  |  |
| Exterior = | 22938.000 | square feet |  |  |  |  |
| Calculation: |  |  |  |  |  |  |
| C = | 7.4805 | gal/ft3 |  |  |  |  |
|  | Thickness (inches) | Coverage Area (ft2) | Transfer Efficiency (%) | Emission Factor (VOC lbs/gal | Total lbs |  |
| Interior coating | 0.012 | 8630.740 | 65 | 1.399 | 138.958 |  |
| Exterior primer/protector | 0.006 | 22938.000 | 65 | 0.179 | 23.626 |  |
| Exterior – sealer/finisher | 0.004 | 22938.000 | 65 | 1.173 | 103.217 |  |
| Total |  |  |  |  | 265.801 |  |
| *Coating will take 24 days – 265.801 ÷ 24 days = 11.075 lbs per day |  |  |  |  |  |  |